July 19, 1991

ACCENT USER'S MANUAL ADDENDUM

ACCENT-SX ADDENDUM

Accent-SX is a plug-in Accent card for Toshiba's T1200XE and

T2000/SX/SXe laptops.  It plugs directly into the Built-in Modem

slot of these computers.  Accent-SX functions exactly the same as

Accent-XE and has the same circiut board as Accent-XE with

minor modifications for fitting into the computers.  It uses the

same Device Driver (SPKEMS.DVC, V4.2 or higher) as Accent-mini

and supports all T1200XE and T2000/SX/SXe features.  Due to the

complexity of the installation process, Accent-SX will be

manufacturer installed ONLY.

ACCENT-L40 ADDENDUM

Accent-L40 is a plug-in Accent card for IBM's laptop model PS/2

L40SX.  It plugs directly into the Internal Fax/Modem slot of this

computer.  This card is built with low-power CMOS circuits for

extremely low power consumption.  The speech output comes

directly from the laptop's system speaker if an external speaker or

headset is not connected.  It uses the same Device Driver

(SPKEMS.DVC, V4.2 or higher) as Accent-mini and supports all

PS/2 L40SX features.

INSTALLATION

(1) Accent-L40's Device Driver, SPKEMS.DVC, should be installed

into the IBM PS/2 L40SX following the same procedures as for

Accent-mini.

(2) Power off the computer and unscrew the keyboard and top

cover.  Plug the connector into the corresponding 50 pin female

connector on the computer's mother board and securely tighten

Accent-L40 to the L40SX's mother board with the original screw.

(3) Run a power test for Accent-L40 by plugging the PS/2 L40SX's

AC adaptor in to ensure power to the computer, and power it on;

Accent-L40 should say, "Accent Ready." through the L40SX's

system speaker after the Device Driver is loaded.

(4) Power off the computer and reassemble.

Accent User's Manual, Version 4.0, July 19, 1991.

Note:  This is the diskette version of the Accent User's Manual which

is generated from the print version.  There is no variation in content.

WARRANTY REGISTRATION

To validate your product warranty, please complete and return this

Warranty Registration within 30 days of purchase.

Name                                       Title

Company                                    Department

Address

City                 State/Country         Zip

Daytime phone (        )                   Date purchased

Product purchased

Serial number

Store Name/Phone                           (        )

Store Address

Store City           State/Country         Zip

Help Us Help You! Your answers to this brief questionnaire will help

us meet your needs.

1.  What type of computer is your Accent connected to?

a.      IBM PC/XT/AT, or compatible   Model #

b.      IBM PS/2, or compatible   Model #

c.      Toshiba Laptop Model #

d.      Other (specify)

2.  What is your application for Accent?

3.  What software packages will be used with Accent?

4.  How did you first learn about Accent?

a.      Magazine ad/article   d.      Dealer/Computer Store

b.      Mail literature       e.      Company approved list

c.      Associate/co-worker   f.      Other (Specify)

Thank you for your cooperation!

First printing, December 1986

Second printing, April 1987

Third printing, November 1988

Fourth printing, April 1989

Fifth printing, February 1991

Copyright 1986,1987,1988,1989,1991 by Aicom Corporation.

All rights reserved.

Aicom Corporation believes the information in this publication is

accurate as of its publication date; such information is subject to

change without notice.  Aicom Corporation is not responsible for any

inadvertent errors.

Aicom is a registered trademark of Aicom Corporation.

Accent is a registered trademark of Aicom Corporation.

IBM is a registered trademark of International Business Machine

Corporation.

IBM PC is a trademark of International Business Machine

Corporation.

PS/2 computer is a trademark of International Business Machine

Corporation.

Microchannel computer is a trademark of International Business

Machine Corporation.

Toshiba is a registered trademark of Toshiba International Corporation.

ACCENT USER'S MANUAL

(VERSION 4.0)

Accent-PC     -      IBM PC/XT/AT & Compatibles

Accent-MC     -      IBM Microchannel PS/2s

Accent-SA     -      Stand-Alone for all computers

Accent-mini   -      IBM PC/XT/AT & Compatibles

Accent-1600   -      Toshiba T1600, T1200 & Compatibles

Accent-XE     -      Toshiba T1000SE

Aicom Corporation

1590 Oakland Road,  Suite B112

San Jose, CA  95131

Tel:  (408)  453-8251    Fax:  (408)  453-8255

FEDERAL COMMUNICATION COMMISSION RADIO FREQUENCY INTERFERENCE

STATEMENT

(Applies to Accent-PC and Accent-SA)

WARNING : This equipment complies with the limits for a Class

B computing device in accordance with the specifications in

Subpart J of Part 15 of the FCC rules.  These specifications are

designed to minimize radio frequency interference in a residential

installation.  However, there is no guarantee that radio or

television interference will not occur in any particular installation.

If this equipment does cause interference to radio or television

reception, which can be determined by turning the equipment off

and on when the radio or television is on, the user is encouraged

to try to correct the interference by one or more of the following

measures:

(1)    Reorient the radio or television receiving antenna.

(2)    Relocate the equipment with respect to the receiver.

(3)    Move the equipment away from the receiver.

(4)    Plug the equipment into a different outlet so that the

equipment and the receiver are on different circuits.

If necessary, the user should consult the dealer or an experienced

radio/television technician for additional suggestions.  The user

may find the following booklet prepared by the Federal

Communications Commission helpful:

"How to Identify and Resolve Radio/TV Interference Problems"

This booklet is available from the U.S. Government Printing Office,

Washington, DC 20402 by ordering stock number 004-000-00345-4.

TABLE OF CONTENTS

SECTION       TITLE                                  PAGE

1             ABOUT THE MANUAL

1.1           INTRODUCTION TO ACCENT                 1 -   2

PRODUCTS

1.2           STANDARD FEATURES FOR ALL              1 -   3

ACCENT MODELS

1.3           DESCRIPTIONS OF MODELS                 1 -   4

1.3.1         ACCENT-PC                              1 -   4

1.3.2         ACCENT-MC                              1 -   5

1.3.3         ACCENT-SA                              1 -   6

1.3.4         ACCENT-mini                            1 -   7

1.3.5         ACCENT-1600                            1 -   8

1.3.6         ACCENT-XE                              1 -   9

2             INSTALLATION

2.1           INSTALLING ACCENT-PC                   2 -   1

2.1.1         SWITCH SETTINGS                        2 -   1

2.1.2         JUMPER SETTINGS                        2 -   2

2.1.3         INSTALLING ACCENT'S SOFTWARE           2 -   4

DEVICE DRIVER

2.1.4         INSTALLING ACCENT-PC                   2 -   5

HARDWARE

2.1.5         SPEAKER CONNECTION                     2 -   5

2.1.6         RUNNING THE DEMO PROGRAM               2 -   5

2.1.7         EXTERNAL POWER SUPPLY FOR              2 -   6

STAND-ALONE OPERATION

2.2           INSTALLING ACCENT-MC                   2 -   6

2.2.1         JUMPER SETTINGS                        2 -   7

2.2.2         INSTALLING ACCENT-MC                   2 -   7

SOFTWARE DEVICE DRIVER

2.2.3         INSTALLING ACCENT-MC                   2 -   7

HARDWARE

2.3           INSTALLING ACCENT-SA                   2 -   7

2.3.1         RS232C CONNECTION                      2 -   9

2.3.2         SPEAKER CONNECTION                     2 -   9

2.3.3         POWERING UP ACCENT-SA                  2 -   9

2.3.4         RUNNING THE DEMO PROGRAM               2 -  10

2.4           INSTALLING ACCENT-mini                 2 -  10

2.4.1         JUMPER SETTINGS                        2 -  12

2.4.2         INSTALLING ACCENT-mini                 2 -  12

SOFTWARE DEVICE DRIVER

2.4.3         INSTALLING ACCENT-mini                 2 -  12

HARDWARE

2.5           INSTALLING ACCENT-1600                 2 -  12

2.5.1         INSTALLING ACCENT-1600                 2 -  12

SOFTWARE DEVICE DRIVER

2.5.2         INSTALLING ACCENT-1600                 2 -  13

HARDWARE

2.6           INSTALLING ACCENT-XE                   2 -  13

2.6.1         INSTALLING ACCENT-XE                   2 -  13

SOFTWARE DEVICE DRIVER

2.6.2         INSTALLING ACCENT-XE                   2 -  13

HARDWARE

3             OPERATION

3.1           INTERFACE TO ACCENT                    3 -   1

3.1.1         DEVICE DRIVER INTERFACE                3 -   1

3.1.2         RS-232C INTERFACE                      3 -   3

3.2           SOFTWARE COMMANDS                      3 -   5

3.2.1         SYSTEM OPTION COMMANDS                 3 -   6

3.2.1.1       Reset                                  3 -   6

3.2.1.2       Initialize                             3 -   6

3.2.1.3       Flush                                  3 -   7

3.2.1.4       RS-232C Echo On                        3 -   7

3.2.1.5       RS-232C Echo Off                       3 -   7

3.2.1.6       Enable Appending <CR> for Status Reply 3 -   7

3.2.1.7       Disable Appending <CR> for Status Reply3 -   8

3.2.1.8       Enable Speech Synthesizer              3 -   8

3.2.1.9       Disable Speech Synthesizer             3 -   8

3.2.1.10      Speech Pause                           3 -   8

3.2.1.11      Speech Continue                        3 -   9

3.2.1.12      Host Input Time-out                    3 -   9

3.2.1.13      Enable <CR> as Alternative             3 -   9

Start-Speaking Character

3.2.1.14      Disable <CR> as Alternative             3 - 10

Start-Speaking Character

3.2.1.15      Enable CTL-Q/S as Xon/Xoff Characters   3 - 10

3.2.1.16      Disable CTL-Q/S as Xon/Xoff             3 - 10

Characters

3.2.1.17      Enable Fast Processing for Speech       3 - 11

3.2.1.18      Disable Fast Processing for Speech      3 - 11

3.2.1.19      Enable DCD Verbal Message               3 - 11

3.2.1.20      Disable DCD Verbal Message              3 - 11

3.2.1.21      Enable Delayed Process for Speech       3 - 12

Option Commands

3.2.1.22      Disable Delayed Process for Speech      3 - 12

Option Commands

3.2.1.23      Enable Verbal Stress on                 3 - 12

Single-Character Alphabets

3.2.1.24      Disable Verbal Stress on                3 - 12

Single-Character Alphabets

3.2.1.25      Set Audio Output Volume                 3 - 13

3.2.1.26      Increase Overall Volume by One Step     3 - 13

3.2.1.27      Decrease Overall Volume by One Step     3 - 13

3.2.1.28      Increase Average Pitch by One Step      3 - 13

3.2.1.29      Decrease Average Pitch by One Step      3 - 14

3.2.1.30      Increase Speech Rate by One Step        3 - 14

3.2.1.31      Decrease Speech Rate by One Step        3 - 14

3.2.1.32      Increase Voice Characteristic by One Step3 - 14

3.2.1.33      Decrease Voice Characteristic by One Step3 - 14

3.2.1.34      Increase Space Pause Time by One Step   3 - 15

3.2.1.35      Decrease Space Pause Time by One Step   3 - 15

3.2.1.36      Increase Sentence Intonation by One     3 - 15

Step

3.2.1.37      Decrease Sentence Intonation by One     3 - 15

Step

3.2.1.38      Enable PC to RS-232C Transmission       3 - 15

3.2.1.39      Disable PC to RS-232C Transmission      3 - 16

3.2.1.40      Command String Sent to Auxiliary Port   3 - 16

3.2.1.41      Command String Not Sent to Auxiliary    3 - 16

Port

3.2.1.42      Status Reply to Host Only               3 - 16

3.2.1.43      Status Reply to Auxiliary Port Only     3 - 17

3.2.1.44      Status Reply to Host and Auxiliary Port 3 - 17

3.2.1.45      Enable LPT Auxiliary Device Chained     3 - 17

Operation

3.2.1.46      Disable LPT Auxiliary Device Chained    3 - 17

Operation

3.2.1.47      Set PC Host                             3 - 18

3.2.1.48      Set RS-232C Host                        3 - 18

3.2.1.49      Enable CTL-X as the Instant Flush       3 - 18

Command

3.2.1.50      Disable CTL-X as the Instant Flush      3 - 19

Command

3.2.1.51      Enable Xon-Always-Sent after Flush      3 - 19

3.2.1.52      Disable Xon-Always-Sent after Flush     3 - 19

3.2.1.53      Enable Index Marker Auto-Prompt         3 - 19

3.2.1.54      Disable Index Marker Auto-Prompt        3 - 20

3.2.1.55      Enable Foot Switch Status Reply         3 - 20

3.2.1.56      Disable Foot Switch Status Reply        3 - 20

3.2.2         SPEECH OPTION COMMANDS                  3 - 20

3.2.2.1       Text Mode                               3 - 21

3.2.2.2       Spell Mode                              3 - 21

3.2.2.3       Punctuation Not Spoken                  3 - 21

3.2.2.4       Punctuation Spoken                      3 - 21

3.2.2.5       <-> Not Spoken                          3 - 22

3.2.2.6       <-> Spoken as DASH                      3 - 22

3.2.2.7       <-> Spoken as MINUS                     3 - 22

3.2.2.8       Enable Abbreviation Dictionary          3 - 22

3.2.2.9       Disable Abbreviation Dictionary         3 - 22

3.2.2.10      Full Set Punctuation Spoken             3 - 23

3.2.2.11      Subset Punctuation Spoken               3 - 23

3.2.2.12      <A> and <a> Always Spoken as /EI/       3 - 23

3.2.2.13      <A> and <a> are Context Dependent       3 - 23

3.2.2.14      Set Average Pitch                       3 - 23

3.2.2.15      Set Voice Characteristic                3 - 24

3.2.2.16      Set Speech Rate                         3 - 24

3.2.2.17      Set Sentence Level Intonation           3 - 24

3.2.2.18      Set Space Pause Time                    3 - 25

3.2.2.19      Set Number Processor                    3 - 25

3.2.2.20      Direct Parameter Input to Synthesizer   3 - 26

Chip

3.2.2.21      Fixed Abbreviation Pronunciation        3 - 27

3.2.2.22      Automatic Abbreviation Pronunciation    3 - 27

3.2.3         STATUS REQUEST COMMANDS                 3 - 27

3.2.3.1       Reply End of Speech                     3 - 28

3.2.3.2       Reply Synthesizer Status                3 - 28

3.2.3.3       Verbal Reply Version Number             3 - 28

3.2.4         INDEX MARKER COMMANDS                   3 - 29

3.2.4.1       Enable CTL-F as the Index Marker        3 - 30

3.2.4.2       Disable CTL-F as the Index Marker       3 - 30

3.2.4.3       Reply <ESC>Y plus Index-Marker          3 - 30

Character for Auto-Prompt

3.2.4.4       Reply Index-Marker Character only for   3 - 30

Auto Prompt

3.2.4.5       Enable Index-Marker-Count Replying      3 - 31

for Status Reply

3.2.4.6       Disable Index-Marker-Count Replying     3 - 31

for Status Reply

3.2.4.7       User Defined Index-Marker               3 - 31

3.2.5         TABLE - 1 (Command Summary)             3 - 32

4             STANDARD ACCENT

TEXT PROCESSOR

4.1           PRONUNCIATION RULES                    4 -   1

4.1.1         ABBREVIATIONS                          4 -   3

4.1.2         ALTERNATIVE PRONUNCIATIONS             4 -   4

4.1.3         MONETARY $ AMOUNTS                     4 -   4

4.1.4         NUMERALS                               4 -   5

4.1.5         TIME OF DAY                            4 -   6

4.1.6         WORDS WITH NO VOWEL                    4 -   6

4.2           PHONEME INPUT                          4 -   7

4.2.1         TABLE-2 (Accent Phoneme List)          4 -   7

5             STANDARD ACCENT

DCD UTILITY

5.1           DCD SOFTWARE OPERATION                 5 -   1

5.2           RULES FOR BUILDING THE                 5 -   2

CUSTOM DICTIONARY

5.2.1         THE ROOT WORD                          5 -   3

5.2.2         THE PART-OF-SPEECH                     5 -   4

5.2.3         WORDS WITH TWO                         5 -   4

PRONUNCIATIONS

5.2.4         A SAMPLE DCD SOURCE FILE               5 -   5

5.2.5         ERROR MESSAGES                         5 -   5

6             PROGRAMMING

ACCENT

7             SPECIFICATIONS

7.1           ACCENT-PC SPECIFICATIONS               7 -   1

7.2           ACCENT-MC SPECIFICATIONS               7 -   2

7.3           ACCENT-SA SPECIFICATIONS               7 -   3

7.4           ACCENT-mini SPECIFICATIONS             7 -   3

7.5           ACCENT-1600 SPECIFICATIONS             7 -   4

7.6           ACCENT-XE SPECIFICATIONS               7 -   5

LIST OF TABLES

TABLE-1       Command Summary                         3 - 32

TABLE-2       Accent Phoneme List                    4 -   7

LIST OF FIGURES

FIGURE-1      Accent-PC Hardware                     2 -   3

FIGURE-2      Accent-SA Back Panel                   2 -   8

FIGURE-3      Accent-mini Hardware                    2 - 11

1. ABOUT THE MANUAL

It is extremely important for the user to read this manual

before trying to operate Accent, since the manual provides

accurate, complete information for ALL ACCENT MODELS

regarding installation, system options, and operation.  The manual

also provides a guide to applications.

This manual has seven Sections.

Section 1 serves as the general introduction to Accent products

(models).

Section 2 tells how to connect each specific Accent model for

operation.

Section 3 details Accent's hardware and software operation, and is

particularly important for engineers or advanced users

wishing to write application programs.

Section 4 describes the standard Accent text processor; the specific

pronunciation rules are covered as well as the phoneme input

definitions.

Section 5 describes the operation for the Downloadable Custom

Dictionary (DCD) utility for advanced users.

Section 6 provides hints and techniques for programming Accent

products in general applications.

Section 7 gives the specifications of each model.

It is suggested the user read through Section 1 for all Accent

models, then jump to model dependent sections from Section 2 on.

The <> are used throughout this manual to bracket an ASCII

character when confusion might occur, for example:

<ESC>         Represents the Escape character

<CR>          Represents the Carriage Return character

<LF>          Represents the Line Feed character

<SP>          Represents the Space character

<HT>          Represents the Horizontal Tabulation character

<->           Represents the Hyphen character

<ESC>WC       Represents the Escape, W, and C character

A table of contents, list of tables and list of figures have been

provided at the beginning of this manual.  Numbered paragraphs

and subparagraphs allow for easy reference.

1.1           INTRODUCTION TO

ACCENT PRODUCTS

Accent speech synthesizer is a high quality text-to-speech

convertor.  It converts ordinary ASCII text into intelligible speech.

Accent's unlimited vocabulary capability, plus its many feature

options, comprise a useful voice-output device for various

applications.

To cover a wide range of the system configurations, cost

requirements, and environmental requirements, six different

Accent models are available to the user.  These six models are:

Accent-PC for PC plug-in; Accent-MC for IBM microchannel PS/2

plug in; Accent-SA for stand-alone operation; Accent-mini for PC

plug-in; Accent-1600 for Toshiba laptop models T1100-plus, T1200,

T1600, T3100SX, and Toshiba portable models T3100-20, T3100E,

T5100, and T5200; and Accent-XE for Toshiba laptop model

T1000SE.

For the PC/PS2 and Toshiba plug-in models, the text and

commands (three-character escape sequences) are sent to Accent

through a re-entrant device driver provided, while the stand-alone

model through an RS232C serial port with either hardware or

Xon/Xoff protocol.

All Accent models employ the standard Accent text-processor which

can speak in two modes:  the text mode and the spell mode.

In the text mode, Accent will convert text into ordinary speech,

employing grammatical rules for parsing phrases and sentences,

for words having two pronunciations, for abbreviations, $ amounts,

numbers, and special signs (#,%,& and so on).  In the text mode,

Accent also employs punctuation marks for sentential intonations.

Through software commands, speech parameters such as average

pitch, Accent's voice characteristic, and speech rate, among others,

can be programmed.  In the spell mode, all text including

punctuation will be spoken letter by letter.

Accent's standard vocabulary is larger than the 20,000 most

commonly used words with precise pronunciation.  For special,

customized vocabulary, a standard DCD (Downloadable Custom

Dictionary) utility is provided, in addition to the phoneme input

capability.  The DCD hardware is built in for all Accent models.

1.2           STANDARD FEATURES

FOR ALL ACCENT

MODELS

Standard Accent text-processor; vocabulary over the 20,000

most commonly used words with precise pronunciation.

Four groups of software commands; system option

commands, speech option commands, status request

commands, and index marker commands.

Speech rate can be adjusted from about 60 to 800 words

per minute.

Control-X character as instant speech-flush command.

Average response time less than 20 milliseconds in Fast-

processing mode.

Demo/Utility diskette including DCD development

software.

DCD hardware built in.

On-board audio power amplifier with volume control; 0.5

Watt.

3.5mm miniature phone jack for 8-Ohm speaker, 4-Ohm

speaker, or headphones.

1.3           DESCRIPTIONS OF

MODELS

The following sections describe the model dependent features for

all six different Accent models.  Throughout this manual, the

general term "Accent" represents all Accent models.

1.3.1         ACCENT-PC

Accent-PC is a self-sufficient, single-board microcomputer with a

640-byte, wrap-around text buffer.  It can be plugged directly into

any IBM PC, PC/XT, PC/AT or compatibles.

A standard RS-232C serial port is also provided for interface with

a computer, terminal, or printer.

The user can select either the PC or the RS-232C as Accent-PC's

host.  Text and software commands are sent from the host to

Accent-PC for processing by an on-board processor.

FEATURES

On-board microprocessor, 640-byte wrap-around text buffer.

IBM PC bus compatible; I/O addresses switch selectable.

Standard RS-232C serial port as a DCE device; baud rate,

parity and data bits switch selectable.

Power connector provided for stand-alone operation or

system integrations.

3k-byte re-entrant device driver provides PC interface

either through DOS (device name) or BIOS (LPT port)

requiring no CPU time overhead or any hardware interrupt

from the PC for the real-time speech processing.

SETACNT.EXE utility program for changing Accent-PC's

device name, I/O addresses, auxiliary device (LPT) number,

and chained-LPT operations.

1.3.2         ACCENT-MC

Accent-MC is a self-sufficient, single board microcomputer, text-to-

speech convertor that can be plugged directly into IBM's

microchannel PS/2 computers.  All functions are performed through

the microchannel, therefore no DIP switch settings are necessary.

The speech output comes directly from the PS/2's system speaker

if an external speaker or headphones is not connected.  It uses the

same Device Driver as Accent-PC.

FEATURES

Functionally compatible with Accent-PC and 60% faster

response than Accent-PC.

On-board microprocessor, 2k-byte text buffer.

Two 3.5mm audio phone jacks on board; one mono, for an

external speaker and one stereo, for external headphones.

Adaptor Descriptive File provided for system configuration.

A foot pedal switch is offered as an option.

1.3.3         ACCENT-SA

Accent-SA is the stand-alone version of the Accent family.  It is a

self-sufficient, microcomputer subsystem that is fully battery

operatable with a 2k-byte wrap-around text buffer housed in a

compact metal case.  For battery conservation, this board is built

with low-power CMOS devices.

A standard RS-232C serial port is provided for interface with a

computer or terminal as the host.  A wall-mount AC adaptor is

provided for external power.

FEATURES

Self sufficient, stand-alone text-to-speech convertor.

Battery operatable; fully charged battery tested over 15

hours of continuous use without recharge.

2k-byte wrap-around text buffer.

Standard RS-232C serial port as a DCE device; baud rate,

parity, and data bits switch selectable; hardware protocol

as default, Xon/Xoff protocol may be selected via software

command.

Wall-mount AC adaptor included.

Internal speaker for added portability.

Two 3.5mm audio phone jacks on board; one mono, for an

external speaker and one stereo, for external headphones.

One thumb-wheel as power on/off switch and volume

control.

1.3.4         ACCENT-mini

Accent-mini consists of a half-length plug-in card and a re-entrant

software device driver for the desktop IBM PC and compatibles.

For battery conservation, when used with various portable

computers, this board is built with low-power CMOS devices.

This model fully supports EMS 3.2 or higher.  The memory

requirement for the device driver is 128k bytes of expanded

memory and only 14k bytes of conventional memory.  If the

amount of expanded memory available is only 64k bytes, Accent-

mini's device driver will utilize 64k bytes of expanded memory and

64k bytes of conventional memory.  If the EMS is not installed

prior to Accent-mini's device driver, 121k bytes of conventional

memory will automatically be used.

Accent-mini's utility program SETMINI.EXE is a powerful tool for

users to set Accent-mini's system parameters and configurations,

such as changing the Device Name, selecting the I/O addresses and

Hardware Interrupt, assigning Accent-mini to any LPT number as

an auxiliary device, setting the chained operation for multiple

devices using the same Interrupt, etc.

FEATURES

Realtime text-to-speech synthesis; hardware interrupt

driven.

2k-byte wrap-around internal phoneme buffer.

SETMINI.EXE utility program for changing Accent-mini's

device name, I/O addresses, hardware interrupts, auxiliary

device (LPT) number, chained operations, etc.

IBM PC bus compatible; I/O addresses and Interrupt

jumpers selectable in conjunction with SETMINI.EXE

utility.

All commands and interface format compatible with

Accent-PC device driver.

Two 3.5mm audio phone jacks on board; one mono, for an

external speaker and one stereo, for external headphones.

Thumb-wheel type volume control.

1.3.5         ACCENT-1600

Accent-1600 is a plug-in Accent card for the Toshiba laptop models,

T1100-plus, T1200, T1600 and T3100SX and Toshiba portable

models, T3100-20, T3100E, T5100, and T5200.  The board plugs

directly into the Internal Modem slot of these computers, making

this unit as portable as the laptop or portable computer.  For

battery conservation, this board is built with low-power CMOS

devices.  This model fully supports all the features for the above

computers and uses the same device driver as Accent-mini.

FEATURES

Functionally compatible with Accent-mini.

Internal speaker for added portability.

Two 3.5mm audio phone jacks on board; one mono, for an

external speaker and one stereo, for external headphones.

Thumb-wheel type volume control.

1.3.6         ACCENT-XE

Accent-XE is a plug-in Accent card for the Toshiba laptop model

T1000SE.  The board plugs directly into the Built-in Modem slot

of the T1000SE.  For battery conservation, this board is built with

low-power CMOS devices.  It fully supports all T1000SE features

and uses the same Device Driver as Accent-mini.

FEATURES

Functionally compatible with Accent-mini.

Internal speaker for added portability.

Two 3.5mm audio phone jacks on the board; one mono, for

an external speaker and one stereo, for external

headphones.

Thumb-wheel type volume control.

2. INSTALLATION

The installation for each specific model may be carried out by the

user and is described in the following sections.

2.1           INSTALLING ACCENT-PC

This Section describes step-by-step how to install the Accent-PC in

the PC for standard operation, and how to connect to an external

power supply for stand-alone operation (custom system integrations

only).  When setting up the hardware, please refer to Figure-1 which

shows the locations of the switches, jumpers, and connectors on

Accent-PC's printed circuit board.

2.1.1         SWITCH SETTINGS

SW1 SETTINGS

SW1 is used to set up the RS-232C baud rate, the parity bit and

the length of the data bits.  Factory default is set to select 9600

baud rate, parity disabled, and 7 data bits.  To change the baud

rate, parity option or data bit length, please refer to Section 3.1.2.

The default settings for SW1 have positions 4 and 8 ON and

positions 1, 2, 3, 5, 6, and 7 OFF.

SW2 SETTINGS

SW2 is used to assign the PC's I/O addresses to Accent-PC.

Factory default addresses are set to 2A8-2AB Hex, which will not

conflict with any PC, PC/XT, or PC/AT standard or reserved I/O

device.  The default settings for SW2 have positions 1, 3, 5, 7 ON

and 2, 4, 6, 8 OFF. Should these default addresses conflict with

any custom built I/O device or if there is to be more than one

Accent-PC installed in a single PC, please refer to the following

paragraphs for appropriate address settings, and to Section 3.1.1

for changing the same addresses in Accent-PC's device driver.

Four consecutive I/O addresses of the PC bus are possessed by each

Accent-PC.  The address always starts with an even number as a

multiple of four.  The starting address may be selected by setting

SW2 (location shown in Figure 1).  SW2 is an 8 position switch,

each position (starting from 1) corresponding to an address bit

(starting from 0) of the PC bus is listed as follows:

SW2 POSITION:                1     2     3     4     5     6     7     8

ADDRESS BIT:     A0    A1    A2    A3    A4    A5    A6    A7    A8    A9

When a SW2 position is set to "ON", its corresponding PC address

bit is set to "0", and vice versa.  "A0" and "A1" are ignored when

setting up SW2; they are "0" for calculating the starting address.

2.1.2         JUMPER SETTINGS

J2 SETTINGS

J2 is used to select Accent-PC's host.  Factory default setting

selects the PC as host.  To select the PC as host, pin 1 and pin 2

of J2 should be jumpered together, using the jumper header

provided.  To select the RS-232C as host, pin 2 and pin 3 of J2

should be jumpered together.

J4 SETTINGS

J4 is used to connect the Reset signal from the PC bus to

Accent-PC.  Factory default provides jumpered J4.  If J4 is

jumpered, whenever the Reset signal from the PC bus is active

(asserted), Accent-PC will be reset.  If J4 is not jumpered,

Accent-PC will automatically reset itself only during power-up.  To

use external power supply for stand-alone operation, J4 must not

be jumpered.

FIGURE-1 Accent-PC Hardware

(Not Shown)

2.1.3         INSTALLING ACCENT'S

SOFTWARE DEVICE DRIVER

From the Demo/Utility diskette provided, copy the following files

to a system diskette or the hard disk which is to be used with

Accent:

SPKACNT.SYS (COPY to the filename CONFIG.SYS)

SPKACNT.DVC

DEMO.BAS

DEMO.TXT

NOTE: For Accent-mini/1600/XE, COPY SPKEMS.SYS instead of

SPKACNT.SYS to the CONFIG.SYS file and COPY SPKEMS.DVC

instead of SPKACNT.DVC for the device driver.

If there are other device drivers already installed in the system

diskette, the file CONFIG.SYS should reside in the system

diskette.  If this is the case, then instead of copying the file

SPKACNT.SYS or SPKEMS.SYS from the Demo/Utility diskette,

simply add (edit) the following statement(s) in the resident file

CONFIG.SYS :

DEVICE=SPKACNT.DVC          for Accent PC/MC

or

DEVICE=EMM.SYS

DEVICE=SPKEMS.DVC           for Accent-mini/1600/XE

Re-boot the PC with the system diskette, and if the message

"Accent-PC/MC Device Driver (Version 4.0) Installed", or "Accent

EMS Device Driver (Version 4.0) Installed" for Accent-mini/1600/

XE appears on the screen then Accent's device driver has been

successfully installed.  If not, carefully go through the device driver

installation procedures again.

Note: If you did not have the EMM.SYS loaded prior to the

SPKEMS.DVC, or if you did not reserve at least 64k bytes of the

expanded memory for Accent-mini/1600/XE, the statement

"SPKEMS: EMS Manager Not Present or No Available Expanded

Memory" will appear on the screen.  In this case the

SPKEMS.DVC will automatically use 121k bytes of the

conventional memory.

Accent's default device name and auxiliary device name are "SPK"

and "LPT3" respectively.  After the hardware installation,

COPYing any text or text file to either "SPK" or "LPT3" will cause

Accent to speak the COPYed text or text file.  Also after the

hardware installation, every time the Accent device driver is

loaded, Accent will verbally prompt "ACCENT READY.".

Accent's demonstration program DEMO.BAS is written in IBM

PC's BASIC Language, so that the system diskette to be used with

Accent must contain the BASIC Interpreter "BASIC.COM" or

"BASICA.COM".

2.1.4         INSTALLING ACCENT-PC

HARDWARE

After turning off all power to the PC, Accent-PC may be plugged

in the PC.  First, the bottom of the metal bracket on the Accent-PC

board should be slid into the bracket slot located at the edge of

the PC mother board.  Then Accent-PC should be held upright and

pushed symmetrically down into the PC edge connector.  Finally,

install the screw tightly to secure Accent-PC's metal bracket

against the PC frame.

2.1.5         SPEAKER CONNECTION

An external speaker may be connected using a 3.5mm male

miniature phone plug (mono).  An 8-Ohm or 4-Ohm speaker is

recommended.  There is no external speaker needed for Accent-

MC/1600/XE installations.

2.1.6         RUNNING THE DEMO PROGRAM

Only when Accent-PC is properly installed, a speaker has been

connected, and the volume has been turned up to approximately

1/3 (clock-wise), may the PC be powered up.

When the PC prompts, type in "BASIC DEMO" or "BASICA

DEMO" (DEMO232 for Accent-SA) to execute the demonstration

program.  If Accent says "AICOM..." at this point, Accent has been

successfully installed.  If not, turn off the power and go through

the installation procedures again, making sure, among other

things, that the switch settings are correct for your particular

application.

In the DEMO diskette provided, there is a document file named

"READ.ME" which explains the applications of each file provided.

2.1.7         EXTERNAL POWER SUPPLY FOR

STAND ALONE OPERATION

If Accent-PC is to be integrated into a custom system for

stand-alone operation, power to Accent-PC must be provided

through PS1.  PS1 is a 5-pin male power connector (Molex

09-64-1051 compatible).  Any suitable female connector may be

used to plug into PS1.  Power requirements are listed in Section 7.

Please note that stand-alone operation is for advanced

users only.  The Warranty is void for, and does not cover,

stand-alone operation.

Power connections for PS1 are the following:

Pin-1  GND

Pin-2  Key (not used)

Pin-3  -12 volt

Pin-4  +5  volt

Pin-5  +12 volt

Pin-6  Not Used

2.2           INSTALLING ACCENT-MC

This Section describes step-by-step how to install Accent-MC in a

microchannel PS/2.

2.2.1         JUMPER SETTINGS

J1 SETTINGS

J1 is used to connect the Reset signal from the microchannel bus

to Accent-MC.  Factory default provides jumpered J1.  If J1 is

jumpered, whenever the Reset signal from the microchannel bus is

active (asserted), Accent-MC will be reset.  If J1 is not jumpered,

Accent-MC will automatically reset itself only during power-up.

All other jumpers should not be installed for proper operation.

2.2.2         INSTALLING ACCENT-MC

SOFTWARE DEVICE DRIVER

All software installation procedures are identical to those of

Accent-PC except the following:  The first step in the software

installation is to reconfigure your PS/2 with Accent-MC's Adapter

Descriptive File, named @6152.ADF, from the Accent Demo/Utility

diskette.  Refer to Section 2.1.3 to install the software device

driver.

2.2.3         INSTALLING ACCENT-MC

HARDWARE

After turning off all power to the PS/2, Accent-MC may be plugged

into the PS/2.  Slide Accent-MC into any slot of your PS/2 machine

and install the screws tightly to secure the board.

After hardware installation is complete, refer to Section 2.1.6 to

run the Demo Program.

2.3           INSTALLING ACCENT-SA

This Section describes how to connect Accent-SA to the AC

adaptor, speaker, and RS-232C for operation.  When setting up the

hardware, please refer to Figure-2 which shows Accent-SA's back

panel.

FIGURE-2 Accent-SA Back Panel

(Not Shown)

2.3.1         RS-232C CONNECTION

Accent-SA's RS-232C interface is configured as a DCE device.

Please refer to Section 3.1.2 if software Xon/Xoff handshaking is

used.

SW1 is used to set up the RS-232C baud rate, the parity bit and

the length of the data bits.  Factory default is set to select 9600

baud rate, parity disabled, and 7 data bits.  To change the baud

rate, parity option and data bit length, please refer to Section

3.1.2.  The default settings for SW1 have positions 4 and 8 ON,

and positions 1, 2, 3, 5, 6, and 7 OFF.

2.3.2         SPEAKER CONNECTION

An external speaker may be connected using a 3.5mm male

miniature phone plug (mono) or  external headphones using a

3.5mm male miniature phone plug (stereo).  An 8-Ohm or 4-Ohm

speaker is recommended.

2.3.3         POWERING UP ACCENT-SA

Turn on the Accent-SA power switch only after the following have

been done:

The Accent-SA is properly set up and connected to a host.

An external speaker or external headsets are attached, if

desired.

The AC adaptor is connected to Accent-SA and plugged into

the wall.

The volume has been turned up to approximately 1/3

(clock-wise).

Accent's power-on message, "ACCENT READY.", should be heard.

If the message is not heard, turn off the power and carefully go

through the installation procedures again.

2.3.4         RUNNING THE DEMO PROGRAM

Accent-SA may be demonstrated with an IBM PC.  First, connect

a straight-through, 25-pin RS-232C cable from the PC's COM1 to

Accent-SA's RS-232C port.  Second, from the Accent Demo/Utility

diskette provided, copy the DEMO232.BAS and DEMO.TXT files

to a system diskette containing IBM PC's BASIC Interpreter

"BASIC.COM" or "BASICA.COM".  Then go to Section 2.1.6 to run

the Demo program.

2.4           INSTALLING ACCENT-mini

This Section describes step-by-step how to install Accent-mini in

the PC.  If your PC does not have any non-standard card, the

default settings of Accent-mini's hardware and software device

driver should function properly without any change.

If your PC has any non-standard card, which uses I/O addresses

3EE-3EF Hex or hardware interrupt IRQ2/IRQ9 (IRQ2 is named

for the PC/XT, and IRQ9 is named for the PC/AT), please refer to

Figure-3 to resolve any conflict, and to Section 3.1.1 to change the

same parameters in Accent-mini's device driver before the

installation.

When setting up the hardware, please refer to Figure 3 which

shows the locations of the jumpers and connectors on Accent-mini's

printed circuit board.

FIGURE-3 Accent-mini Hardware

(Not Shown)

2.4.1         JUMPER SETTINGS

J1 SETTINGS

J1 is used to select the PC's hardware interrupt for Accent-mini.

Factory default setting selects IRQ2/IRQ9 (IRQ2 for the PC/XT, or

IRQ9 for the PC/AT).

J2 SETTINGS

J2 is used to assign PC I/O addresses to Accent-mini.  Two

consecutive I/O addresses are used by each Accent-mini.  Factory

default addresses are set to 3EE-3EF Hex.

2.4.2         INSTALLING ACCENT-mini

SOFTWARE DEVICE DRIVER

The DOS version 2.11 or higher is required to run Accent-mini.

With a system diskette containing DOS-2.11 or higher, go to

Section 2.1.3 to install the software device driver.

2.4.3         INSTALLING ACCENT-mini

HARDWARE

Refer to Section 2.1.4 to install Accent-mini's hardware.  After the

hardware installation is complete, refer to Sections 2.1.5 and 2.1.6

to run the Demo Program.

2.5           INSTALLING ACCENT-1600

This Section describes step-by-step how to install Accent-1600 into

your Toshiba laptop or portable computer.

2.5.1         INSTALLING ACCENT-1600

SOFTWARE DEVICE DRIVER

Utilize the Toshiba computer's utility program to reserve 128k

bytes of the expanded memory for Accent-1600.  Refer to Section

2.1.3 to install the software device driver.

2.5.2         INSTALLING ACCENT-1600

HARDWARE

After turning off all power to the Toshiba computer, Accent-1600

may be installed into the computer's Internal Modem slot.

Carefully plug the Accent-1600 board into the Internal Modem slot,

and tightly install any necessary screws to secure Accent-1600.

Turn Accent-1600's volume up to 1/3, clock-wise.

After the hardware installtion is complete, refer to Section 2.1.6 to

run the Demo Program.

2.6           INSTALLING ACCENT-XE

This Section describes step-by-step how to install Accent-XE into

your Toshiba T1000SE laptop.  Accent-XE comes with a custom

drilled Built-in-Modem cover for the laptop.

2.6.1         INSTALLING ACCENT-XE

SOFTWARE DEVICE DRIVER

First, utilize the T1000SE's utility program to reserve 128k bytes

of the expanded memory for Accent-XE.  Refer to Section 2.1.3 to

install the software device driver.

2.6.2         INSTALLING ACCENT-XE

HARDWARE

After turning off all power to the T1000SE, Accent-XE may be

installed into the laptop's Built-in Modem slot.  Carefully plug

Accent-XE into the Built-in Modem slot and slowly move it up and

down so that Accent-XE's female connector fits into the T1000SE's

male connector.  Tightly install the two screws to secure the

Accent-XE board.  Turn Accent-XE's volume up to 1/3.

After the hadware installation is complete, refer to Section 2.1.6 to

run the Demo Program.

3. OPERATION

This Section describes Accent's user-interface and software

command options.  The hardware setup during the installation will

define Accent's interface options, and the software commands will

allow users to change the system options, and to request status

from Accent.

The host computer communicates with Accent as a peripheral

device either via the software device driver or via the RS-232C

port.  If Accent is used with a software device driver on a PC, the

PC BUS protocol is automatically handled in the device driver.

The user-interface to the software device driver is explained in

Section 3.1.1.

If the RS-232C interface is used, the handshaking for the RS-232C

can either be hardware (Default) or Xon/Xoff type.  The RS-232C

interface is described in Section 3.1.2.

3.1           INTERFACE TO ACCENT

The following two sections (Section 3.1.1 through 3.1.2) are model

dependent, the user may refer to the appropriate sections for any

specific model.

3.1.1         DEVICE DRIVER INTERFACE

This section is not for Accent-SA.  A device driver is used so that

an application program (usually written in a High Level Language)

may communicate with a specific device such as a disk drive, a

printer, or a COM port.  If the system diskette used to boot the PC

has a device driver in it, the PC will automatically load the device

driver during boot time.  Each device driver has its specified name.

Accent's default device name is SPK.

After the device driver has been loaded into the PC, any

application program can OPEN or CLOSE the device as a disk file,

READ and WRITE may be performed to that device.  The device

names are reserved (can't be used elsewhere) by the system once

they are installed.

Since Accent is a special device (not a standard device, such as a

standard PC printer), a custom-built device driver is needed to

allow the user to access Accent, that is, to OPEN, CLOSE, READ

and WRITE Accent like a standard device with a specific name.

In this case, the name of Accent's character device driver is SPK.

Please refer to IBM PC DOS Technical Reference manual for

detailed descriptions of Installable Device Drivers.  Accent's device

driver is a character device driver, and it provides the functions

of INIT, INPUT, and OUTPUT as described in the DOS Technical

Reference.

Although Accent's device drivers are re-entrant, the Calls to DOS

will not be re-entrant.  Therefore an alternative entry is provided

-- the LPT port.  Since Calls to LPT port (by using INT 17H) is

through the BIOS, which is re-entrant, only Calls from LPT to

Accent will be re-entrant.  The default LPT port for Accent is set

to LPT3.  The following shows the LPT interface definitions:

FUNCTION:     Print Character

ENTRY:        Same as IBM BIOS

RETURN:       Same as IBM BIOS

NOTE:         None

FUNCTION:     Initialize

ENTRY:        Same as IBM BIOS

RETURN:       Same as IBM BIOS

NOTE:         Accent will be Flushed.  This includes all previous

inputs, text and commands. FOR A QUICK

FLUSH, USE THIS FUNCTION.

FUNCTION:     Status Read

ENTRY:        Same as IBM BIOS

RETURN:       Same as IBM BIOS for AH bit-0, and bit-3 to bit-7.

AH bit-2,1 = 00 for not end-of-speech.

AH bit-2,1 = 01 for complete end-of-speech.

AH bit-2,1 = 10 for near end-of-speech (about 0.3

to 2 second to end.)

DH = Index-marker-count (only if <ESC>YZ

command has been previously issued).

NOTE:         With the index marker feature enabled, if a

constant status-check loop is performed to the

device driver LPT BIOS call, Status Read, the loop

time-period is recommended to be at least 20

milliseconds.

FUNCTION:     Accent's Foot Switch Status Read (For Accent-MC

Version 4.0 or higher only).

ENTRY:        AH = 12 Hex

RETURN:       AL = Foot Switch Status;

Bit 0 = Not used

Bit 1 = 1 for Fast-forward (Left) switch being

pressed

Bit 2 = 1 for Play (Center) switch being

pressed

Bit 3 = 1 for Rewind (Right) switch being

pressed

NOTE:         If a constant status-check loop is performed on the

Foot Switch, the loop time period is recommended

to be at least 20 milliseconds.

If Factory default setups for the system parameters and

configurations are not desired, the utility program SETACNT.EXE

for Accent-PC/MC, or SETMINI.EXE for Accent-mini/1600/XE, may

be used to change them.  Please refer to the menu when running

the SETACNT.EXE or SETMINI.EXE.

3.1.2         RS-232C INTERFACE

This section is only for Accent-PC/SA.  Accent's RS-232C is a

standard serial interface port for Data Communication Equipment

(DCE).  If hardware hand-shaking is adopted, the RTS, DTR, CTS

and DSR signals must be used/checked for proper operation.  If

software Xon/Xoff handshaking is adopted, the software command

<ESC>=V must be sent to Accent first after power-up for proper

operation.  After Accent has received the <ESC>=V command, the

CTS and DSR signals will be asserted by Accent at all times, and

the RTS and DTR signals from the host will not be checked by

Accent.

To ensure hardware engineers of a clear understanding of the

RS-232C signals, each signal on the 25-pin RS-232C D-connector

is listed below for reference.

25-PIN               I/O           RS-232C        RS-232C

CONNECTOR            TO/FROM       SIGNAL         SIGNAL

PIN NUMBER           ACCENT        NAME           SOURCE

1             BOTH          PGND           BOTH

2             INPUT         TXD            DTE

3             OUTPUT        RXD            DCE

4             INPUT         RTS            DTE

5             OUTPUT        CTS            DCE

6             OUTPUT        DSR            DCE

7             BOTH          SGND           BOTH

20             INPUT         DTR            DTE

22             OUTPUT        RING           DCE

The RS-232C options include baud rate, parity, and data bit

selections. SW1 (refer to FIGURE-1 for Accent-PC, and FIGURE-2

for Accent-SA) is used to set these options, as follows:

BAUD RATE

SW1 POSITION         1      2      3      4       5

75                   ON     OFF    OFF    OFF     ON

150                  OFF    ON     OFF    OFF     ON

300                  ON     OFF    OFF    OFF     OFF

600                  OFF    ON     OFF    OFF     OFF

1200                 OFF    OFF    ON     OFF     ON

2400                 OFF    OFF    OFF    ON      ON

4800                 OFF    OFF    ON     OFF     OFF

9600                 OFF    OFF    OFF    ON      OFF

PARITY

SW1 POSITION         6      7

NONE                 OFF    OFF

ODD                  ON     OFF

EVEN                 ON     ON

The data bit length is set up by SW1 position 8: ON for 7 data bits,

OFF for 8 data bits.  The stop bit is fixed at 1 bit.  Factory default

settings for these options are: 9600 baud, no parity, and 7 data

bits.

3.2           SOFTWARE COMMANDS

Once Accent hardware has been set up, operation is completely

under control of the host.  Accent can accept two kinds of data

from its host: text (including phoneme input), and commands.  Text

may be translated into speech and commands will control Accent's

operation.

In the text mode, input text from the host is stored in Accent's text

buffer and will not be spoken until a sentence punctuation mark

is received, the time-out described in Section 3.2.1.12 is

encountered, or a <CR> is received if the option in Section 3.2.1.13

is enabled.  Section 4 describes Accent's text processing rules in

detail.  In the spell mode, the text input will be spelled out

character by character immediately after it is received.

Commands are sent from the host to Accent as escape sequences.

An escape sequence is a string of ASCII characters preceded by an

<ESC> character (1B Hex and 27 decimal).  There are four groups

of commands: System option commands, speech option commands,

status request commands, and index marker commands.

An Accent command consists of the <ESC> character immediately

followed by two more characters.  Unless otherwise noted, the

speech option commands and the status request command

<ESC>?S will not be executed until all text, previously sent from

the host, has been processed, that is, until Accent's internal text

buffer has been emptied.  Other commands will be executed

immediately upon receipt of the command.  Illegal Escape

Sequences SHOULD NOT be sent to Accent.

Each command is described in detail in the following sections.  A

Commands Summary table (Table-1), which also gives Accent's

software default settings, is located in Section 3.2.5.  In these

commands, the upper and lower cases are distinguished.  The

model dependent commands are noted in the descriptions.

3.2.1         SYSTEM OPTION COMMANDS

Various system option commands are available for different

application configurations.

3.2.1.1       Reset

<ESC>=R

Reset Accent, equivalent to power up reset.  If the RS-232C is used

for host interface, allow at least two seconds for Accent to complete

its self-diagnostics.  No data should be sent to Accent during this

two-second period or the data will be lost.  "ACCENT READY."

verbal message will be issued by Accent.  All options will be reset

to Accent's default state.

3.2.1.2       Initialize

<ESC>=X

Reset Accent, equivalent to power up reset but with no power-on

verbal message replied.  If the RS-232C is used for host interface,

allow at least 100 milli-seconds for Accent to complete its self-

diagnostics.  No data should be sent to Accent during this 100

milli-seconds period or the data will be lost.

3.2.1.3       Flush

<ESC>=x

All Accent's internal text/speech buffers will be flushed and speech

will be stopped immediately, all options and status remain

unchanged.  This command is most frequently used to flush the

speech.  Important notice:  Also refer to Section 3.2.1.49 through

3.2.1.51 for Control-X character as instant Flush command and for

Xon-always-sent after Flush.

3.2.1.4       RS-232C Echo On

<ESC>=E

This command is only for Accent-PC/SA.  Characters received from

Accent's RS-232C will be echoed back to the host device

immediately.

3.2.1.5       RS-232C Echo Off

<ESC>=e

This command is only for Accent-PC/SA.  Characters received from

Accent's RS-232C will not be echoed back to the host device.  This

is the power-up default state.

3.2.1.6       Enable Appending <CR> for Status

Reply

<ESC>=C

The reply to the Status Request Commands in Section 3.2.3 will

have a <CR>, Carriage Return, appended at the end.  This

command is useful for application software written with

High-Level Languages.  This is the power-up default state.

3.2.1.7       Disable Appending <CR> for Status

Reply

<ESC>=c

The reply to the Status Request Commands in Section 3.2.3 will

not have a <CR> appended at the end.

3.2.1.8       Enable Speech Synthesizer

<ESC>=A

Text received from the host will be converted to speech.

3.2.1.9       Disable Speech Synthesizer

<ESC>=a

Text received from the host will not be converted to speech and

will not be stored in the text buffer either.  All other options are

unaffected.

3.2.1.10      Speech Pause

<ESC>=Q

Speech will be stopped (paused) immediately, or after the current

syllable is spoken if Accent is speaking.  Further text input from

the host will be stored in the text buffer and not spoken until a

Speech Continue Command described in Section 3.2.1.11 is

received.  If Accent is in the Speech Pause state, only the Speech

Continue, Reset, Initialize, Flush, Set Average Pitch, Set Speech

Rate, and Set Voice Characteristic commands will be accepted.

3.2.1.11      Speech Continue

<ESC>=q

Accent will continue to speak, beginning where speech was stopped

(paused) by the Speech Pause Command described in Section

3.2.1.10.

3.2.1.12      Host Input Time-out

<ESC>Tn

This command is only for Accent-PC/MC/SA.  The n in this escape

sequence represents any digit 0 through 9, which determines the

time-out period while Accent awaits text input from the host.  The

time-out period ranges approximately from 0.04 seconds to 20

seconds for digits 0 through 9.  Any higher digit doubles the

time-out period of the preceding lower digit.

With any occurrence of time-out, if there is text in the text buffer

with no end-of-sentence punctuation or a terminating <CR> (if the

option in Section 3.2.1.13 is enabled), Accent will begin to speak

out the text.  Default time-out is set to 7, which gives a time-out

period of about 5 seconds.

3.2.1.13      Enable <CR> as Alternative

Start-Speaking Character

<ESC>=F

Accent will start speaking when a <CR> is received.  This

command is useful for Screen-Review applications for the visually

impaired.  However, it may slightly affect sentence intonation.

3.2.1.14      Disable <CR> as Alternative

Start-Speaking Character

<ESC>=f

Accent will start speaking only when a sentence punctuation mark

is received, or the host input time-out is encountered.  This is the

power-up default state.

3.2.1.15      Enable CTL-Q/S as Xon/Xoff

Characters

<ESC>=V

This command is only for Accent-PC/SA.  Xon/Xoff software hand-

shaking will be turned on for the RS-232C.  The Xon character is

Control-Q (11 Hex) and the Xoff character is Control-S (13 Hex).

IMPORTANT NOTE: If Xon/Xoff is to be used, this command

should be sent to Accent immediately after power-up.  After Accent

has sent an Xoff to the host, there are still 128 bytes free space for

input.  If further inputs exceed the 128-byte limit, buffer overrun

will occur.  For Accent-PC, Accent's host must be set to RS-232C

before this command can be issued.

3.2.1.16      Disable CTL-Q/S as Xon/Xoff

Characters

<ESC>=v

This command is only for Accent-PC/SA.  Accent's Xon/Xoff

software handshaking will be turned off.  The RS-232C protocol is

controlled by the RTS, DTR, CTS, and DSR hardware signals.

This is the power-up default state.

3.2.1.17      Enable Fast Processing for Speech

<ESC>=B

If Accent's speech buffer is empty, Accent will process the input

text immediately after a phrase or a word is received.  This

command is mainly used in applications requiring fast response

time such as in the Screen-Review applications.  Note that if this

option is enabled with high speech rate, the sentence intonation

will be affected.

3.2.1.18      Disable Fast Processing for Speech

<ESC>=b

Accent will process the text when a complete sentence is received.

This is the normal and power-up default state.

3.2.1.19      Enable DCD Verbal Message

<ESC>=J

Upon completion of a DCD download, Accent will deliver a verbal

message of "Custom Dictionary Download Completed."  This is the

power-up default state.

3.2.1.20      Disable DCD Verbal Message

<ESC>=j

Upon completion of a DCD download, Accent will not deliver any

verbal message.  This command may be used if more than one set

of DCD is to be used while Accent is active.

3.2.1.21      Enable Delayed Process for Speech

Option Commands

<ESC>=K

Speech option commands will only be processed after all text,

previously sent from the host, has been processed.  This is the

normal and power-up default state.

3.2.1.22      Disable Delayed Process for Speech

Option Commands

<ESC>=k

Speech option commands will be processed immediately after they

are received.  This command is used only when an instant speech

parameter change (such as pitch, speech rate, etc.) is required.

3.2.1.23      Enable Verbal Stress on

Single-Character Alphabets

<ESC>=U

Every single-character alphabet will be verbally stressed when

spoken by Accent.  This command is useful when Accent is reading

a string of alphabets and/or mixed with digits such as a serial

number.

3.2.1.24      Disable Verbal Stress on

Single-Character Alphabets

<ESC>=u

Normal text-processing rule will be employed to pronounce

single-character alphabets.  Verbal stress/non-stress is context

dependent.  This is the power-up default state.

3.2.1.25      Set Audio Output Volume

<ESC>An

The n in this escape sequence represents any digit 0 through 9,

which determines the overall volume of the audio output.  The

digit 9 has the highest volume which is the default state.  Unless

definitely needed for special effect, this parameter should be left as

9 for the best resolution.  The volume is recommended to be

controlled by the volume control knob/thumbwheel.

3.2.1.26      Increase Overall Volume by One

Step

<ESC>+A

The overall volume will be increased by one step instantaneously.

NOTE: All <ESC>+ and <ESC>- related commands will be

processed instantly, and if the parameter has reached its upper or

lower limit, it will stay at its limit.  These zoom features are useful

for locating the most comfortable voice for the user.

3.2.1.27      Decrease Overall Volume by One Step

<ESC>-A

The overall volume is decreased by one step.

3.2.1.28      Increase Average Pitch by One Step

<ESC>+P

The average pitch is increased by one step.

3.2.1.29      Decrease Average Pitch by One Step

<ESC>-P

The average pitch is decreased by one step.

3.2.1.30      Increase Speech Rate by One Step

<ESC>+R

The speech rate is increased by one step.

3.2.1.31      Decrease Speech Rate by One Step

<ESC>-R

The speech rate is decreased by one step.

3.2.1.32      Increase Voice Characteristic by One

Step

<ESC>+V

The voice characteristic is increased by one step.

3.2.1.33      Decrease Voice Characteristic by One

Step

<ESC>-V

The voice characteristic is decreased by one step.

3.2.1.34      Increase Space Pause Time by One

Step

<ESC>+S

The space pause time is increased by one step.

3.2.1.35      Decrease Space Pause Time by One

Step

<ESC>-S

The space pause time is decreased by one step.

3.2.1.36      Increase Sentence Intonation by One

Step

<ESC>+M

The sentence level intonation is increased by one step.

3.2.1.37      Decrease Sentence Intonation by One

Step

<ESC>-M

The sentence level intonation is decreased by one step.

3.2.1.38      Enable PC to RS-232C Transmission

<ESC>=P

This command is for Accent-PC only.  Characters received from PC

will immediately be sent to Accent-PC's RS-232C port.  Refer to

Section 3.2.1.40 and 3.2.1.41 for command character string.

3.2.1.39      Disable PC to RS-232C Transmission

<ESC>=p

This command is for Accent-PC only.  Characters received from PC

will not be sent to the RS-232C port.

3.2.1.40      Command String Sent to Auxiliary

Port

<ESC>=L

This command is for Accent-PC only.  Command character string

(the three-character escape sequence) will be sent to the auxiliary

port if the transmission path is enabled through Section 3.2.1.38.

Note: If PC is set to be Accent-PC's host, then the RS-232C port is

considered to be Accent's auxiliary port, and vise versa.  This

command has no effect if Accent-PC is set to RS-232C host.

3.2.1.41      Command String Not Sent to Auxiliary

Port

<ESC>=l

This command is for Accent-PC only.  Command character string

will not be sent to the auxiliary port whether or not the

transmission path is enabled through Section 3.2.1.38. This is the

power-up default state.

3.2.1.42      Status Reply to Host Only

<ESC>=G

This command is for Accent-PC only.  The reply to the Status

Request Commands in Section 3.2.3 will be sent to the host only.

This is the power-up default state.

3.2.1.43      Status Reply to Auxiliary Port Only

<ESC>=g

This command is for Accent-PC only.  The reply to the Status

Request Commands in Section 3.2.3 will be sent to the auxiliary

port only.

3.2.1.44      Status Reply to Host and Auxiliary

Port

<ESC>=H

This command is for Accent-PC only.  The reply to the Status

Request Commands in Section 3.2.3 will be sent to the host and to

the auxiliary port.

3.2.1.45      Enable LPT Auxiliary Device

Chained Operation

<ESC>=Z

The description of this escape sequence is not for Accent-SA.

Characters received from LPT auxiliary device will be processed by

Accent, and the same characters will also be sent to the original

software interrupt vector (by a FAR JUMP) previously saved by

the device drivers during system-boot.  For example, if the LPT

number is set to 1 (by SETACNT.EXE or SETMINI.EXE) and the

LPT chain flag is set, all text sent to Accent via LPT1 or PRN will

be spoken by Accent, and will also be sent to the standard system

printer to be printed.

3.2.1.46      Disable LPT Auxiliary Device

Chained Operation

<ESC>=z

This command is not for Accent-SA.  Characters received from LPT

auxiliary device will be processed by Accent only and will not be

sent to the standard system printer.

3.2.1.47      Set PC Host

<ESC>HP

This command is for Accent-PC only.  Set the PC as Accent-PC's

host.  When Accent-PC is first powered up, the host is selected by

Hardware J2 as described in Section 2.1.2.  This command allows

users to change the host and utilize the auxiliary port for

communication with a second host or with a peripheral, as desired.

Note: when Accent-PC's host is changed by a set-host command, all

options and status in Accent-PC remain valid, except that they will

then be referred to the newly designated host.

3.2.1.48      Set RS-232C Host

<ESC>HS

This command is for Accent-PC only.  It sets the RS-232C as

Accent-PC's host.

3.2.1.49      Enable CTL-X as the Instant Flush

Command

<ESC>=M

This command will enable the Control-X character (18 Hex) as the

instant Flush command.  After this command has been issued to

Accent, both the <ESC>=x and the Control-X will cause Accent to

flush.  The Control-X will flush instantly since it is a single-

character command.

3.2.1.50      Disable CTL-X as the Instant Flush

Command

<ESC>=m

Only the <ESC>=x command will cause Accent to flush.  This is

the power-up default state.

3.2.1.51      Enable Xon-Always-Sent after Flush

<ESC>=I

This command is only for Accent-PC/SA.  If Accent is in Xon/Xoff

protocol mode, this command will cause Accent to always send an

Xon to the host after Accent has been Flushed by the host.  This

Xon is mainly used to signal the host that all index marker

Auto-Prompts (as described in Section 3.2.1.53) have been sent to

the host.

3.2.1.52      Disable Xon-Always-Sent after Flush

<ESC>=i

This command is only for Accent-PC/SA.  If Accent is in Xon/Xoff

protocol mode, Accent will send an Xon to the host after a Flush

only if an Xoff has been previously sent to the host.  This is the

power-up default state.

3.2.1.53      Enable Index Marker Auto-Prompt

<ESC>=Y

This command is only for Accent-PC/SA.  This command will

enable Accent to automatically prompt each index marker to the

host via the RS-232C, when Accent has spoken to the position of

each index marker.

3.2.1.54      Disable Index Marker Auto-Prompt

<ESC>=y

This command is only for Accent-PC/SA.  No index marker will be

prompted automatically.  This is the power-up default state.

3.2.1.55      Enable Foot Switch Status Reply

<ESC>=W

This command is for Accent-MC/SA Version 4.0 or higher.  This

command enables the foot switch status to be replied whenever a

Status Request command is sent; refer to Section 3.2.3 for the

replied information of the foot switch.  This option allows for the

control of such functions as fast forward, rewind, and resume.

If a constant loop is used for the status request, a delay is

recommended for each loop.  The recommended delay time is the

same as the Status Request (refer to Section 3.2.3).

3.2.1.56      Disable Foot Switch Status Reply

<ESC>=w

This command is only for Accent-MC/SA Version 4.0 or higher.

The foot switch status will not be replied when a Status Reply

command is sent, refer to Section 3.2.3.  This is the power-up

default state.

3.2.2         SPEECH OPTION COMMANDS

This command group applies to all Accent models, it allows the

user to tailor Accent to speak text and punctuation marks in a

number of desired formats according to the requirements of a

specific application.  Furthermore, the speech rate, the average

(baseline) pitch, the voice characteristic, the Space pause time,

sentence level intonation, as well as the internal abbreviation

dictionary and schwa sound manipulations can be programmed

through these commands.

3.2.2.1       Text Mode

<ESC>OT

Text from the host will go through Accent's text processor for

speech output.  This is the normal text-to-speech operation.

3.2.2.2       Spell Mode

<ESC>Ot

Text from the host will be spoken character by character (i.e.,

spelled out) by Accent.  This includes all special signs and

punctuation marks.  For listing, please refer to Section 4.1.

3.2.2.3       Punctuation Not Spoken

<ESC>Op

No punctuation mark will be spoken in the Text Mode.  This is the

normal text-to-speech operation.

3.2.2.4       Punctuation Spoken

<ESC>OP

Punctuation marks will be spoken as described in Section 3.2.2.10

and Section 3.2.2.11.  This command may be used, for example, in

applications where the punctuation marks must be spoken out

precisely, such as in reading a program written in a High-Level

Language.

3.2.2.5       <-> Not Spoken

<ESC>Om

The <-> sign will not be spoken by Accent.  This is the normal

text-to-speech operation and the power-up default state.

3.2.2.6       <-> Spoken as DASH

<ESC>OM

The <-> sign will be spoken as DASH by Accent.

3.2.2.7       <-> Spoken as MINUS

<ESC>ON

The <-> sign will be spoken as MINUS by Accent.

3.2.2.8       Enable Abbreviation Dictionary

<ESC>OI

Accent's internal abbreviation dictionary will be searched during

text-to-speech rule processing.  This is the power-up default state.

3.2.2.9       Disable Abbreviation Dictionary

<ESC>Oi

Accent's internal abbreviation dictionary will not be searched

during text-to-speech rule processing.

3.2.2.10      Full Set Punctuation Spoken

<ESC>OR

If Punctuation Spoken command is enabled as in Section 3.2.2.4,

all punctuation marks will be spoken as described in Section 4.1.

This is the power-up default state.

3.2.2.11      Subset Punctuation Spoken

<ESC>Or

If Punctuation Spoken command is enabled as in Section 3.2.2.4,

only ,.:;!? punctuation marks will be spoken as described in Section

4.1.

3.2.2.12      <A> and <a> Always Spoken as /EI/

<ESC>OA

The character <A> and <a> will always be pronounced as /EI/

phoneme rather than context dependent.

3.2.2.13      <A> and <a> Pronunciation are

Context Dependent

<ESC>Oa

The character <A> will always be pronounced as /EI/ phoneme,

while the character <a> is context dependent.  In most cases, the

<a> will be pronounced as the schwa phoneme /UU/.

3.2.2.14      Set Average Pitch

<ESC>Pn

The n in this escape sequence represents any digit, 0 through 9,

which determines the average pitch level of Accent.  The range of

the average pitch is from about 70Hz to 140Hz for digits 0 through

9.  The nominal and default pitch level for Accent is 5, which gives

an average pitch of about 115Hz.

3.2.2.15      Set Voice Characteristic

<ESC>Vn

The n in this escape sequence represents any digit, 0 through 9,

which determines the characteristic of Accent's voice.  The lower

numbers will give lower and deeper sounding voices.  The higher

numbers will give higher and sharper sounding voices.  Usually,

when the average pitch is set at a lower number, the voice

characteristic will also be set at a lower number to match, and vice

versa.

However, for special applications such as Games, this parameter

may be set to any number and, in conjunction with other Speech

Option Commands, will provide two thousand voices representing

different personalities.  The default number for this parameter is

set to 5 which will give a male voice of average tone.

3.2.2.16      Set Speech Rate

<ESC>Rn

The n in this escape sequence represents any digit, 0 through F (in

Hex), G, and H, which determines the speed of Accent's speech

(speech rate).  With the Space Pause Time set to 0, the speech rate

runs from about 120 words per minute to about 800 words per

minute for those 18 digits from 0 to H.

3.2.2.17      Set Sentence Level Intonation

<ESC>Mn

The n in this escape sequence represents digits 0 through 4.  If n

is 0, the regular (full) intonation will be employed, and if n is 1, a

robotic monotone pitch (using the average pitch level set by the

command described in Section 3.2.2.14), ranging from 60 Hz to 150

Hz, will be employed.

If n is 2, 3, or 4, the regular intonation is employed yet the

sentence level intonation is reduced with respect to n=0, while n=4

has the least sentence level intonation reduction, and n=2 the most

sentence level intonation reduction.

For high speech rate output, for example, in Screen-Review

applications, the reduced intonation (or the monotone) will provide

a more comfortable listening.  For proof-reading type of

applications, the reduced intonation is also useful.  This command

applies to both the text mode and the spell mode.

3.2.2.18      Set Space Pause Time

<ESC>Sn

The n in this escape sequence represents any digit, 0 through 9,

which determines the pause time for the Space character between

two words. Lower numbers represent shorter pauses, and higher

numbers represent longer pauses.  The default is set to 0, which

gives the most natural sounding speech.

3.2.2.19      Set Number Processor

<ESC> Nn

The n in this escape sequence represents any digit, 0, 1, and 2,

which determines Accent's number processing level.  Accent's

number processor is described in Section 4.1.3 and 4.1.4.

For n=0, Accent's number processor is turned on.  If a word has

number(s) mixed in it, the word portion will be spelled out

automatically.  This is the power- up default state.

For n=1, Accent's number processor is turned on.  If a word has

number(s) mixed in it, the word portion will be spoken as a word.

This mode is most commonly used in the Screen-Review

applications for faster speech delivery.

For n=2, Accent's number processor is turned off.  All numbers will

be spoken digit by digit.  If a word has number(s) mixed in it, the

word portion will be spoken as a word.

3.2.2.20      Direct Parameter Input to Synthesizer

Chip

<ESC> |~..~

This command is for advanced users only.  It allows the host to

send parameters directly to the synthesizer chip on the Accent

board.  The synthesizer chip on the Accent board is the AI901.

The .. in this escape sequence represents the parameters (in ASCII

Hex numbers) to be sent to the synthesizer chip.  The direct

Parameter Input may be terminated at any time with a ~ (tilde)

character.  The Interrupt will be handled internally.  To use this

feature, the parameters must meet the following conditions.  Refer

to the chip's specification for parameter definitions.

Use upper case Hex numbers only: 0-9, A-F.

Only the Phoneme Response and the Transitioned Pitch

modes are allowed.

The 5-byte parameters should be sent in reverse order:  the

Filter-Frequency byte first, the Phoneme and Duration byte

last.

The lower nibble of any given byte is sent first.

Upon receiving the first 5-bytes of the parameters (10

characters), Accent will start speaking.

The parameters sequence may be infinitely long.

3.2.2.21      Fixed Abbreviation Pronunciation

<ESC>OL

This command is only for the abbreviated words designated by note

2 in the abbreviation table in Section 4.1.1.  If this command is set,

an abbreviated word designated by note 2 will always be

pronounced in the non-note 2 form.

3.2.2.22      Automatic Abbreviation Pronunciation

<ESC>Ol

This command is only for the abbreviated words designated by note

2 in the abbreviation table in Section 4.1.1.  If this command is set,

any abbreviated word will automatically be pronounced according

to the abbreviation pronunciation rules contained in Section 4.1.1.

This is the power-up default state.

3.2.3         STATUS REQUEST COMMANDS

There are four commands for the host to request specific status

from Accent. If a constant status-check loop is performed to Accent

via the RS-232C port, the loop time-period is recommended to be

at least 50 milli-seconds.

The Foot Switch option is supported only for Accent-SA and

Accent-MC; Version 4.0 or higher.  If the Foot Switch Reply is

enabled through the <ESC>=W command, refer to Section 3.2.3.2

for Accent-SA's reply definition, and refer to Section 3.1.1 for

Accent-MC's reply definition (through the device driver).

The Reply Synthesizer Status command is most frequently used for

checking Accent's speech buffer status.

3.2.3.1       Reply End of Speech

<ESC>?S

When speech has ended, Accent will reply with a capitalized

character <S>.

3.2.3.2       Reply Synthesizer Status

<ESC>?s

This request will be immediately replied with a status-character

by Accent.  If the synthesizer is idle (not speaking, speech ended),

Accent will reply with a capitalized character <S>.  If the

synthesizer is speaking, and is not near to the end, Accent will

reply with a lower-case character <s>.  If the synthesizer is

speaking but near to the end (about 0.3 second to 2 seconds of

speech left, dependent on the speech rate), Accent will reply

with an <M> for more text to have uninterrupted speech.  For

host-interface using the RS-232C, the index-marker-count will be

appended right after the status-character if (and only if) the

index-marker-count replying is enabled as described in Section

3.2.4.5.

If the Foot Switch is enabled, an extra byte will be appended to the

above replied data.  The reply definition is as follows:

Bit 0 = Not Used

Bit 1 = 1 for Fast-Forward (Left) switch being pressed

Bit 2 = 1 for Play (Center) switch being pressed

Bit 3 = 1 for Rewind (Right) switch being pressed

3.2.3.3       Verbal Reply Version Number

<ESC>|1

Accent will speak out its text-processor Version number for

identification.

3.2.4         INDEX MARKER COMMANDS

The index marker feature is mainly used in the Screen-Review

applications.  It allows the application programs to track exactly

which word in a given text Accent has spoken to, so that the

screen cursor (or pointer) movement can be synchronized with the

speech.

Accent can accept two kinds of index markers, the Control-F

character or a user defined escape-sequence.  The Control-F is

recommended since it takes less memory space and less time to

process.  The marker will be treated as a Space character by the

Accent text-processor, and it is recommended to be placed right

after the word to be marked.

If the host-interface is through Accent's RS-232C port, the host has

two methods to obtain Accent's marker status: through the

Auto-Prompt feature or through Accent's Status-Reply command

<ESC>?s.  With the Auto-Prompt feature (Section 3.2.1.53)

enabled, Accent will automatically prompt the index marker

character to the host each time a marked word has been spoken

(also refer to Section 3.2.1.51 and 3.2.1.52).  With the Status-Reply

command enabled for replying the index-marker-count, Accent will

append the index-marker-count (in one-byte binary form) to the

status-character (S, M, or s), and then clear Accent's

index-marker-counter for a new count. Either of the above two

methods may be used, but NOT BOTH at the same time.

If the host-interface is through Accent's device driver, the index

marker is supported only through the LPT BIOS call, Status Read.

With the Status-Reply command enabled for replying the

index-marker-count, on return of the LPT BIOS call, Status Read,

the DH register contains the index-marker-count (in one-byte

binary form), and Accent's index-marker-counter is cleared for a

new count.  Refer also to Section 3.1.1.

3.2.4.1       Enable CTL-F as the Index Marker

<ESC>YX

Accent will treat the Control-F character (06 Hex) as the index

marker.

3.2.4.2       Disable CTL-F as the Index Marker

<ESC>Yx

Accent will not treat the Control-F character (06 Hex) as the index

marker.  This is the power-up default state.

3.2.4.3       Reply <ESC>Y plus Index-Marker

Character for Auto-Prompt

<ESC>YY

This command is only for Accent-PC/SA.  The <ESC>Y plus the

index- marker character (one character) will be sent to the host via

the RS-232C port when Accent is prompting the index marker in

the Auto-Prompt mode.  This three-character prompt is used only

if the host needs an escape-sequence for index marker prompt.

3.2.4.4       Reply Index-Marker Character only

for Auto-Prompt

<ESC>Yy

This command is only for Accent-PC/SA. Only the index-marker

character (one character) will be sent to the host via the RS-232C

port when Accent is prompting the index marker in the

Auto-Prompt mode.  This is the power-up default state.

3.2.4.5       Enable Index-Marker-Count Replying

for Status Reply

<ESC>YZ

For host interface using the RS232C, the index-marker-count will

be appended to the replied status for the <ESC>?s command.  For

host interface using the device driver, the index-marker-count will

be returned in the DH register for LPT BIOS call, Status Read.

Note: The index-marker-count should be checked frequently enough

so that it does not overflow.  It is recommended that the time

between two checks should not be over five seconds if every word

in the text has a marker.

3.2.4.6       Disable Index-Marker-Count Replying

for Status Reply

<ESC>Yz

No index-marker-count will be replied.  This is the power-up

default state.

3.2.4.7       User Defined Index-Marker

<ESC>Y_

This three-character escape-sequence is the user defined index-

marker. The <_> character represents the user defined index-

marker character which may be any character A-W, a-w, or 0-9.

Once a <_> is used, it should not be altered.  This user defined

index-marker is used only if the host must use an escape-sequence

as the index-marker.

3.2.5         TABLE-1 (Command Summary)

The following Table summarizes the software commands for all

Accent models.  The model dependent commands are denoted as

follows:

*             For Accent-PC only.

**            For Accent-PC, Accent-SA only.

***           For Accent-PC, Accent-MC, Accent-mini,

Accent-1600, Accent-XE drivers only.

****          For Accent-MC, Accent-SA only.

SEL           Selected for power-up default.

The columns are in the following order: Command, Note, Default,

Description.

System Option Commands

<ESC>=R                     Reset Accent

<ESC>=X                     Initialize Accent per H/W set-up

<ESC>=x                     Flush all internal buffers

<ESC>=E       **            RS-232C echo on

<ESC>=e       **     SEL    RS-232C echo off

<ESC>=C              SEL    Append <CR> for Status Reply

<ESC>=c                     Append no <CR>for Status Reply

<ESC>=A              SEL    Enable speech synthesizer

<ESC>=a                     Disable speech synthesizer

<ESC>=Q                     Speech pause

<ESC>=q              SEL    Speech continue

<ESC>Tn       **     7      Host input time-out; n = 0 to 9

<ESC>=F                     Enable <CR> as alternative

start-speaking character

<ESC>=f              SEL    Disable <CR> as alternative

start-speaking character

<ESC>=V       **            Enable CTL-Q/S as Xon/Xoff char.

<ESC>=v       **     SEL    Disable CTL-Q/S as Xon/Xoff char.

<ESC>=B                     Enable fast processing mode

<ESC>=b              SEL    Disable fast processing mode

<ESC>=J              SEL    Enable DCD verbal message

<ESC>=j                     Disable DCD verbal message

<ESC>=K              SEL    Process Speech Option Commands

only when speech buffer is empty

<ESC>=k                     Process Speech Option Commands

immediately

<ESC>=U                     Verbal stress on single character

<ESC>=u              SEL    No stress on single character

<ESC>An              9      Set Audio Volume; n = 0 to 9

<ESC>+A                     Increase Audio Volume by 1 level

<ESC>-A                     Decrease Audio Volume by 1 level

<ESC>+P                     Increase Average Pitch by 1 level

<ESC>-P                     Decrease Average Pitch by 1 level

<ESC>+R                     Increase Speech Rate by 1 level

<ESC>-R                     Decrease Speech Rate by 1 level

<ESC>+V                     Increase Voice Characteristic by 1

level

<ESC>-V                     Decrease Voice Characteristic by 1

level

<ESC>+S                     Increase Space Pause Time by 1

level

<ESC>-S                     Decrease Space Pause Time by 1

level

<ESC>+M                     Increase Sentence Intonation by 1

level

<ESC>-M                     Decrease Sentence Intonation by 1

level

<ESC>=P       *             Enable PC to RS-232C transmission

<ESC>=p       *      SEL    Disable PC to RS-232C transmission

<ESC>=L       *             Command string sent to Aux port

<ESC>=l       *      SEL    Command string not sent to Aux

port

<ESC>=G       *      SEL    Status Reply to Host only

<ESC>=g       *             Status Reply to Aux port only

<ESC>=H       *             Status Reply to Host and to Aux

port

<ESC>=Z       ***           Enable chained LPT operation for

PC via device driver

<ESC>=z       ***    SEL    Disable chained LPT operation for

PC via device driver

<ESC>HP       *      SEL    Set PC Host

<ESC>HS       *             Set RS-232C Host

<ESC>=M                     Enable CTL-X as the instant Flush

command

<ESC>=m              SEL    Disable CTL-X as the instant Flush

command

<ESC>=I       **            Enable Xon-always-sent after Flush

<ESC>=i       **     SEL    Disable Xon-always-sent after Flush

<ESC>=Y       **            Enable index marker Auto-Prompt

<ESC>=y       **     SEL    Disable index marker Auto-Prompt

<ESC>=W       ****          Enable foot switch status reply

<ESC>=w       ****   SEL    Disable foot switch status reply

Speech Option Commands

<ESC>OT              SEL    Text mode

<ESC>Ot                     Spell mode

<ESC>Op              SEL    Punctuation will not be spoken

<ESC>OP                     Punctuation will be spoken

<ESC>Om              SEL    <-> will not be spoken

<ESC>OM                     <-> will be spoken as DASH

<ESC>ON                     <-> will be spoken as MINUS

<ESC>OI              SEL    Enable Accent's standard

abbreviation table

<ESC>Oi                     Disable Accent's standard

abbreviation table

<ESC>OR              SEL    Full-set punctuation spoken

<ESC>Or                     Subset punctuation (,.;:!?) spoken

<ESC>OA                     <A> or <a> always spoken as /EI/

<ESC>Oa              SEL    <A> always spoken as /EI/, <a> is

spoken as /EI/ or UU/ dependent

on context

<ESC>Pn              5      Average pitch level; n = 0 to 9

<ESC>Vn              5      Voice characteristic; n = 0 to 9

<ESC>Rn              5      Speech rate; n = 0 to F, G, and H

<ESC>Mn              0      Sentence level intonation; n = 0 (full

intonation), 1 (monotone), 2, 3, or 4

<ESC>Sn              0      Space pause time; n = 0 to 9

<ESC>Nn              0      Set number processor n = 0,1,2

<ESC>|~..~                  Direct parameter input to

synthesizer chip

<ESC>OL                     Fixed Abbreviation Pronunciation

<ESC>Ol              SEL    Automatic Abbreviation

Pronunciation

Status Request Commands

<ESC>?S                     Reply <S> when speech has ended

<ESC>?s                     Reply synthesizer status

immediately;

<S> for speech--end,

<s> for speech-not-end,

and <M> speech-near-end

<ESC>|1                     Reply Accent's version verbally

Index Marker Commands

<ESC>YX                     Enable CTL-F as the index marker

<ESC>Yx              SEL    Disable CTL-F as the index marker

<ESC>YY       **            Reply <ESC>Y plus index-marker

character for Auto-Prompt

<ESC>Yy       **     SEL    Reply index-marker character only

for Auto-Prompt

<ESC>YZ                     Enable index-marker-count replying

for Status Reply

<ESC>Yz              SEL    Disable index-marker-count replying

for Status Reply

<ESC>Y_                     User defined index-marker

4. STANDARD ACCENT TEXT PROCESSOR

Accent's text processor is standard for all Accent models.  A set of

legal ASCII characters is defined for Accent's text processor.

Illegal characters will be ignored and not pronounced.  The legal

characters are here listed :

!"#$%&'()*+,-./0123456789:;<=>?@

ABCDEFGHIJKLMNOPQRSTUVWXYZ[\]^_`

abcdefghijklmnopqrstuvwxyz{|}~

plus <SP>, <CR>, <LF>, and <HT>.

The Accent text processor employs a highly sophisticated

text-to-speech algorithm.  Text may be input with or without

capital letters (unless otherwise specified, as for the abbreviations).

The key to, and the only requirement for, attaining natural,

intelligible speech with Accent is that the input text must be

properly punctuated as in normal writing.  Hyphens should not be

omitted in hyphenated words.  Pauses are automatically inserted

into speech by punctuation marks and rules.  If longer pause is

required, consecutive sentence ending punctuation marks (.:;!?)

may be used.  The following Sections describe each pronunciation

rule in detail.

4.1           PRONUNCIATION RULES

Punctuation marks will not be spoken by Accent in normal text

mode unless the option is enabled by the command described in

Section 3.2.2.4.  To insure natural speech from Accent, any word

(or special sign) with a punctuation mark following it may have

only one or no Space character, <SP>, between the word and the

punctuation mark.  Each ASCII character which Accent considers

to be a mark of punctuation is here listed :

SIGN          TYPE                 SPOKEN

<SP>          Word end             Space

,             Phrase end           Comma

(             Phrase begin         Left Paren

[             Phrase begin         Left Bracket

{             Phrase begin         Left Brace

)             Phrase end           Right Paren

]             Phrase end           Right Bracket

}             Phrase end           Right Brace

"             Phrase begin/end     Quote

.             Sentence end         Period

;             Sentence end         Semicolon

:             Sentence end         Colon

?             Sentence end         Question

!             Sentence end         Exclamation

The <`> (grave accent) character is used to mark a stress for its

following syllable in a word.  It may be placed anywhere within a

word which is not an Arabic number, an abbreviation, or a special

sign.  ASCII characters which Accent considers to be special signs

are listed with their pronunciations as follows:

SIGN          SPOKEN               SIGN           SPOKEN

&             And                  *              Star

@             At                   |              Bar

=             Equals               _              Underline

>             Greater              <              Less

#             Number               %              Percent

/             Slash                \              Back Slash

+             Plus

$             Dollar (also see Section 4.1.3)

-             (See Section 3.2.2.5 through 3.2.2.7)

^             Caret (spell mode only)

~             Tilde (spell mode only)

'             Apostrophe (spell mode only)

`             Grave (spell mode only)

<CR>          Return (spell mode only)

<LF>          Line feed (spell mode only)

<HT>          Tab (spell mode only)

<SP>          Space (spell mode only)

4.1.1         ABBREVIATIONS

An abbreviation may be input without a period following it unless

it is at a sentence end or otherwise noted.  A capitalized initial

letter (the first letter) may not be required unless otherwise noted.

Accent's abbreviations list includes the following :

APR.       APRIL (1)          MG.       MILLIGRAMS

APT.       APARTMENT (2)      MGS.      MILLIGRAMS

AUG.       AUGUST             MI.       MICHIGAN

AV.        AVENUE             ML.       MILLILITERS

AVE.       AVENUE             MM.       MILLIMETERS

BLVD.      BOULEVARD          MR.       MISTER

BLDG.      BUILDING           MRS.      MISSES

CA.        CALIFORNIA         MS.       MIZ (2)

CM.        CENTIMETERS        MON.     MONDAY

CMS.       CENTIMETERS        MT.       MOUNT

CO.        COMPANY (2)        NOV.      NOVEMBER

CT.        COURT              NY.       NEW YORK

CU.        CUBIC              OCT.      OCTOBER

DEC.       DECEMBER           OZ.       OUNCES

DEPT.      DEPARTMENT         OZS.      OUNCES

DR         DRIVE              PL.       PLACE

DR.        DOCTOR (2)         PG.       PAGE

EA.        EACH               PPD.      POST PAID

ETC.       ET CETERA          PRES.     PRESIDENT

EXT.       EXTENSION          RD.       ROAD

FEB.       FEBRUARY           REP.      REPRESENTATIVE

FIG.       FIGURE (1)         REV.      REVEREND

FRI.       FRIDAY             SAT.      SATURDAY (2)

FT         FEET               SEC.      SECONDS

FT.        FORT (2)           SECS.     SECONDS

GEN.       GENERAL            SEN.      SENATOR

GM.        GRAMS (1)          SEP.      SEPTEMBER

GOV.       GOVERNOR           SEPT.     SEPTEMBER

HQ.        HEADQUARTERS       SQ.       SQUARE

HR.        HOURS              SR.       SENIOR

HRS.       HOURS              ST        STREET

HZ.        HERTZ              ST.       SAINT (2)

INC.       INCORPORATED       SUN.      SUNDAY (2)

IL.        ILLINOIS           TX.       TEXAS

JAN.       JANUARY (2)        TUE.      TUESDAY

JUL.       JULY               TUES.    TUESDAY

JUN.       JUNE               THU.      THURSDAY

JR.        JUNIOR             THUR.    THURSDAY

KG.        KILOGRAM           THURS.  THURSDAY

KGS.       KILOGRAMS          UNIV.     UNIVERSITY

KM.        KILOMETERS         VOL.      VOLUME

LB.        POUND              VS.       VERSUS

LBS.       POUNDS             WED.     WEDNESDAY (2)

LTD.       LIMITED            YD.       YARD

MAR.       MARCH (2)          YDS.      YARDS

Note (1):     Must have an ending period.

Note (2):     Must have a capitalized initial letter and an ending

period.

Besides the preceding abbreviations, Accent includes hundreds of

other commonly used abbreviations which are normally "spelled

out" (such as EIA, CCITT, CIA, USA, USSR, SOS, etc.) when

pronounced.  Such abbreviations are not listed here.  Note: if there

is no vowel letter in an abbreviated word , it will automatically be

spelled out by Accent.

4.1.2         ALTERNATIVE PRONUNCIATIONS

A special feature of Accent's text processor is that grammatical

rules are applied to words with two pronunciations, and

adjustments are automatic.  If this feature is not desired (such

as in a single-word pronunciation), a <^> immediately following an

ambiguous word will elicit that word's alternative pronunciation.

4.1.3         MONETARY $ AMOUNTS

The following examples show different ways of specifying monetary

amounts, which may be spoken as high as trillion(s) :

Input :       $1

Speech:       One dollar

Input :       $235

Speech:       Two hundred thirty-five dollars

Input :       $.10

Speech:       Ten cents

Input :       $0.10

Speech:       Zero dollar and ten cents

Input :       $1.234

Speech:       One point two three four dollars

Input :       $1.26

Speech:       One dollar and twenty-six cents

Input :       $6723

Speech:       Six seven two three dollars

Input :       $30,456.17

Speech:       Thirty thousand four hundred fifty-six dollars and

seventeen cents

4.1.4         NUMERALS

Any number having five or more digits without a comma inserted

is pronounced digit by digit.  Four-digit numbers without leading

zero(s) will be pronounced as two pairs of two-digit numbers.  If a

number has a comma inserted at every third digit, the following

counting units are available :  Hundred, Thousand, Million, Billion,

and Trillion.  For any number with a decimal point, a leading digit

must be present, otherwise it will be treated as a period.

Following are examples of how Accent pronounces numerals:

Input :       16

Speech:       Sixteen

Input :       1986

Speech:       Nineteen eighty-six

Input :       168

Speech:       One six eight

Input :       0304

Speech:       Zero three zero four

Input :       1,663

Speech:       One thousand six hundred sixty-three

Input :       166-1688-K8

Speech:       One six six, one six eight eight, K eight

Input :       0.123

Speech:       Zero point one two three

Input :       (408)453-8251

Speech:       Four zero eight, four five three, eight two five one

Input :       #123-45-6789

Speech:       Number one two three, four five, six seven eight

nine

4.1.5         TIME OF DAY

Input :       12:45

Speech:       Twelve : forty-five

Input :       7:07

Speech:       Seven : zero seven

4.1.6         WORDS WITH NO VOWEL

If a word has no vowel letter it can not be pronounced, and Accent

will automatically spell it out.  The vowel letters are A E I O U

and Y.  Examples of such words are PM, BC, KWPP, etc.  If a word

containing vowel letters goes through Accent's text processor but

the resulting phoneme sequence has no vowel in it, that word will

also be spelled out.  Examples of such words are NE, YE, etc.

4.2           PHONEME INPUT

The beginning and ending of a phoneme input string to Accent

must be marked by a <~> character.  Immediately following the

beginning mark is a </> character.  In a phoneme input string

sequence, each phoneme is separated by the </> character, and the

sequence must be ended by a </> character.

The Space, Comma, and Period punctuation marks can also be

embedded in a phoneme sequence.  No word should exceed 32

phonemes in length.  The standard phoneme symbols used for

Accent products are listed in TABLE-2.  The following shows an

example of the phoneme input:

Text:         This is a test.

Phoneme:      ~/DH/X/I/S/ /I/Z/ /UU/ /T/'/E/S/T/./~

or            This ~/I/Z/~ a ~/T/'/E/S/T/./~

4.2.1         TABLE-2 (Accent Phoneme List)

PHONEMES                     EXAMPLES

Vowels:

IY                          mEEt

II                          sillY, marY, beautY

I                           sIx, bEfore

IE                          bEfore

E                           bElt

EA                          dAd

AE                          After

UH                          lOve

UU                          About (Schwa sound)

A                           gOt

AA                          whAt

O                           stOre

OW                          Office

U                           lOOk, cOUld

OO                          cartOOn

Diphthongs:

ER                          bIRd

EI                          dAY

OU                          bOAt

AI                          bUY

AU                          hOUse

OI                          bOY

IU                          mUsic, nEW

Consonants:

M                           More

N                           NiNe

NG                          siNGiNG

Y                           Year

W                           Water

R                           Rug, fiRing

RU                          Roof

(Used before phonemes OO and U)

L                           Look

K                           Kit

G                           taG

P                           Put

B                           Buy

T                           siT

D                           DaD

CH                          CHurCH

J                           Just

F                           aFter

V                           loVe

S                           Sit

Z                           Zero

SH                          diSH

ZH                          pleaSure

TH                          wiTH

DH                          THere

H                           Hat

TX                          buTTer

DX                          laDDer

'                           ~/T/'/E/S/T/~

(Word primary stress mark (TEST))

X                           ~/X/A/R/~

(Word secondary stress mark (ARE))

Q                           ~/F/'/AI/R/Q/A/R/M/~

(Morpheme boundary mark

(FIREARM))

5. STANDARD ACCENT DCD UTILITY

The DCD utility is a development software which runs on an IBM

PC for generating the user's own customized dictionary as DCD

files.  If needed, the DCD file can be uploaded from the PC to any

host and then downloaded to Accent by the COPY Command

appropriate to that host's Operating System.  All Accent models

have a built-in, 4k-byte memory block which can hold

approximately 250 10-character words.  Once the DCD is

downloaded, Accent will always search through the DCD first for

any given word during the text-to-speech processing.

5.1           DCD SOFTWARE OPERATION

To generate a custom dictionary for Accent, the DCD development

software (the program named DCD.EXE on the diskette provided)

and a DCD source file are needed.  The DCD source file may be

created (with any Editor for straight ASCII characters) under any

file name with any file extension.

A sample DCD source file is shown in Section 5.2.4.  The followings

are general rules for creating the DCD source file.

(1) Only upper case characters are allowed.

(2) Legal characters for spelling are:

ABCDEFGHIJKLMNOPQRSTUVWXYZ

(3) The spelling, phonemes, and part-of-speech are entered in that

order, and are separated only by two commas.  Phonemes should

conform to the standard Accent phoneme input format as shown in

Section 4.2.1.

(4) The words containing the same first-letter should not exceed

255 in number.  That is, each Alphabetic group should not have

more than 255 words.

(5) No Space character is allowed in the DCD source file.

(6) No empty line is allowed in the DCD source file.

After the DCD source file has been created, the DCD.EXE program

must then be executed (by typing in "DCD xxxxxx.xxx <CR>") to

generate a Hex file and an absolute Object file.  The Hex file will

be used for download.  Both the Hex file and the Object file have

the same name as that of the DCD source file, and the file

extensions are listed as follows:

Hex file:     xxxxxx.DLX

Object file:  xxxxxx.OBJ

During the generation of the DCD, the <.> will be prompted to

indicate the word count.  Each <.> prompt indicates that ten words

have been processed.

After the DCD Hex file and Object file have been generated, the

memory size (in bytes) of the DCD and the error message (if any)

will be shown on the screen.  The user should check to assure that

the memory size does not exceed 4k bytes.

To download the DCD to Accent, use the host computer's COPY

Command to copy the Hex file to Accent through the

communication link, the device driver or the RS-232C port.  Since

each time a DCD file is downloaded to Accent, it overwrites the old

DCD that was in Accent, multiple DCD applications may be

implemented easily.  To totally disable Accent's DCD search after

a DCD file has been downloaded, simply download the

DUMMY.DLX file to Accent.  The DUMMY.DLX file is in the

Demo/Utility diskette provided.

5.2           RULES FOR BUILDING THE

CUSTOM DICTIONARY

Any word in the DCD source file must contain only the capitalized

Alphabetic letters.  Because the Accent Text Processor already

processes numerous prefixes and suffixes, the DCD user will save

memory by entering only the root words, and thereafter testing for

any of their derivatives which he wishes Accent to speak.

The correct part-of-speech for each entry word is required for the

text processor to correctly parse the phrase/sentence and generate

more-natural sounding speech.

5.2.1         THE ROOT WORD

Any specific word which is entered into the DCD will always be

pronounced as phonemically entered.  In addition, that word may

combine with various of its prefixes and/or suffixes to form yet

other words which will also be pronounced correctly.

Nevertheless, due to the complexity of the text processing rules,

internal rule conflicts may result in skewed pronunciation of words

derivative of the root word entered.  Therefore it is suggested that

when any root word is entered into the DCD, all related words for

which the user requires accurate, correct pronunciation be tested

as soon as the DCD has been downloaded. Should a related word

or words be mispronounced (which might happen when, for

example, the word has both a prefix and multiple suffixes), the

user should simply enter the mispronounced word or words into

the DCD.  The Accent text processor supports the following

prefixes and suffixes:

PREFIXES:

ab-           down-         mal-          poly-             sur-

anti-         en-           micro-        post-             sus-

auto-         ex-           mis-          pre-              tele-

be-           grand-        mono-         pro-              trans-

bi-           half-         multi-        re-               tri-

co-           im-           non-          right-            ultra-

com-          in-           out-          semi-             un-

con-          inter-        over-         short-             under-

counter-      ir-           para-         sub-              up-

de-           left-         per-          super-            vice-

dis-

SUFFIXES:

-ic           -ible         -ial          -ern              -est

-d            -wise         -ual          -ship             -ist

-ed           -ate          -ful          -er               -ency

-hood         -uate         -ism          -or               -ly

-ance         -ure          -n            -s                -ally

-ence         -ive          -man          -es               -ery

-side         -ize          -en           -less             -ory

-age          -ing          -ion          -ness             -ty

-like         -ish          -ation        -ous              -ity

-able         -al           -son          -ment

5.2.2         THE PART-OF-SPEECH

Those parts-of-speech which are to be assigned by the DCD user as

he enters vocabulary items, and which are supported by the Accent

text processor, are listed as follows:

PART-OF-SPEECH              REPRESENTATION

1                           Noun

2                           Verb

3                           Adjective

4                           Adverb

7                           Present Participle

8                           Past Participle

13                          Interjection

19                          Verb or Noun (with same

pronunciations)

62                          Verb or Noun/Adjective having -ate

as suffix (with different

pronunciations)

63                          Verb or Noun (with different

pronunciations)

5.2.3         WORDS WITH TWO

PRONUNCIATIONS

If a word has two pronunciations with 63 as its part-of-speech,

both words should be entered into the DCD adjacently.  The first

word should always have the verb pronunciation, with the

part-of-speech as 63.  If a word has two pronunciations with 62 as

its part-of-speech, only the verb needs to be entered into the DCD.

The Accent text processor will search for the correct pronunciation

when parsing the sentence.

5.2.4         A SAMPLE DCD SOURCE FILE

SUPERMAN,S/'/OO/P/ER/M/EA/N/,1

COMBO,K/'/A/M/B/OU/,1

ESTEEM,UU/S/T/'/IY/M/,19

GRADUATE,G/R/'/EA/J/U/EI/T/,62

EAT,'/IY/T/,2

EATING,'/IY/T/I/NG/,7

EATEN,'/IY/T/UU/N/,8

NICELY,N/'/AI/S/L/II/,4

RECORD,R/I/K/'/O/R/D/,63

RECORD,R/'/E/K/ER/D/,1

FINE,F/'/AI/N/,3

5.2.5         ERROR MESSAGES

0      Phoneme name too long, or  missing the phoneme

separator </>

1      Phoneme name not valid

4      Primary or secondary stress not followed by the </>

5      Part of speech not valid

6      Primary or secondary stress disallowed on non-vowel phoneme

7      DCD memory size exceeds 32k-bytes

6. PROGRAMMING ACCENT

The following are some general rules and hints for programming

Accent in various applications:

(1) Once a specific voice (average pitch, speech rate, voice

characteristic, sentence intonation, etc.) has been chosen for

output, the escape sequences along with system option parameters

and DCD (if any) may be sent to Accent as part of power-up

initialization.  The initialization for the Screen-Review applications

is recommended to be <ESC>=F <ESC>=B <ESC>OA <ESC>M4

<ESC>=M <ESC>N1 <ESC>OL.  Other parameters may be user

tailored or selected.

(2) It is important that all text be punctuated normally, and that

compound words be properly hyphenated.  Any word not

pronounced satisfactorily by Accent may be corrected by using the

DCD, phoneme input, or intentional misspelling.

(3) Speech is often better perceived when the pitch is set at a lower

level.  At low pitch, the speech rate may thus be set at a level

higher than usual without losing intelligibility.  From experiments

and listening tests, <ESC>P3 <ESC>V4 settings give an

average-low male voice with very intelligible, natural sounding

speech for regular text at the default speech rate.

(4) Text and escape-sequence commands for any application may

be entered and tested first using the DEMO.BAS (or

DEMO232.BAS) program provided or any suitable Editor.  After

which the text and/or the escape sequences may be saved as a file

for further use or editing.

(5) The <ESC>=F and <ESC>=B commands will affect sentence

intonation.  They are not recommended for general applications or

for Screen-Review applications when slow speech-rate is used for

proof-reading.

(6) If, for the first-time synthetic speech listener, the word

boundaries in a sentence are not perceived distinctly enough, the

Set Space Pause Time command may be used to increase the pause

between words.  Medium short, descriptive type of sentences

always give good comprehension to the listeners, especially when

the listeners are expecting certain prompts.

(7) For applications requiring different voices (by using speech

option commands) as well as using the Flush command frequently,

always try to Flush Accent first then send the speech option

commands.  Enable and use the Control-X character as the Flush

command.

(8) For applications requiring frequent, constant Flush of speech,

unless the application program can assure that Accent's input text

buffer is never full by using the Status Request command in

Section 3.2.3.2, the following recommendations should be taken

into consideration: If Accent's device driver is used for host

interface, the LPT port interface should be adopted because it is

re-entrant, speech may be flushed anytime with a Flush command

(LPT's Initialize command).  If the RS-232C is used for host

interface, the Xon/Xoff protocol should be used because even when

Accent has sent Xoff to the host, it still has 128-byte free space for

further text input or Flush command.

(9) This is for Accent-PC only.  In applications requiring a

multi-host configuration, if the text is both to be spoken and to be

sent to the auxiliary port, it is the user's responsibility to

guarantee that the auxiliary port (either the PC or the RS-232C

port) has an uninterrupted receiving capability so that Accent does

not get hung up waiting for the auxiliary port ready.

7. SPECIFICATIONS

The following sections describe the specifications of each Accent

model.

7.1           ACCENT-PC

SPECIFICATIONS

Parallel Bus:

IBM Personal Computer

Slot:

One full-length slot in IBM PC, PC/XT, PC/AT or

Compatibles

Address:

Switch-selectable

(4 I/O Addresses, Default 2A8-2AB Hex)

Serial I/O:

RS-232C as DCE device

Power:

500 mA Typical @ +5V

600 mA Maximum @ +5V

40 mA Typical @ +12V

80 mA Maximum @ +12V

10 mA Typical @ -12V

20 mA Maximum @ -12V

Load:

1 TTL load/bus line Maximum

Audio Output Power:

0.5W Maximum, 8-Ohm or 4-Ohm Speaker

Physical Dimensions:

5.2" x 10.5" x 0.5" (Maximum component height)

Weight:

0.75 lbs.

Software Device Driver:

DOS-2.11 or higher

Size:  3k bytes

FCC Authorization:

Class-B, Part-15J

7.2           ACCENT-MC

SPECIFICATIONS

Parallel Bus:

IBM PS/2, Microchannel; 8/16 bit I/O card slot

Slot:

One full-length slot in IBM PS/2 Microchannel computer

Address:

Card ID: 6152 Hex

Power:

450 mA Typical @ +5V

550 mA Maximum @ +5V

30 mA Typical @ +12V

80 mA Maximum @ +12V

Load:

1 TTL load/bus line Maximum

Audio Output Power:

0.5W Maximum, 8-Ohm or 4-Ohm Speaker

Physical Dimensions:

3.5" x 11.5" x 0.5" (Maximum component height)

Weight:

0.75 lbs.

Software Device Driver:

DOS-4.01 or higher

Size:  3k bytes

7.3           ACCENT-SA

SPECIFICATIONS

Serial I/O:

RS-232C as DCE device

Power:

9V/300mA AC Adaptor or Internal battery

AC Adaptor Input:

110V/60HzAudio Output Power:

0.5W Maximum, 8-Ohm or 4-Ohm Speaker

Audio Output Power:

0.5W Maximum, 8-Ohm or 4-Ohm Speaker

Physical Dimensions:

5.7" x 8.5" x 1.6"

Weight:

2.5 lbs. (AC adaptor and battery included)

FCC Authorization:

Class-B, Part-15J

7.4           ACCENT-mini

SPECIFICATIONS

Parallel Bus:

IBM Personal Computer

Slot:

One half-length slot in IBM PC, PC/XT, PC/AT, or

Compatibles

Address:

3EE-3EF or 2EE-2EF Hex, jumper-selectable (2 I/O

Addresses; Default 3EE-3EF Hex)

Interrupt:

IRQ2(PC/XT)/IRQ9(PC/AT), IRQ3, IRQ4, or IRQ7;

Jumper-selectable (Default IRQ2/IRQ9)

Power:

All CMOS circuits, minimum power usage

Load:

2 CMOS load/bus line Maximum

Audio Output Power :

0.5W Maximum, 8-Ohm or 4-Ohm Speaker

Physical Dimensions:

4.3" x 4.6" x 0.5" (Max. component height)

Weight:

0.6 lbs.

Software Device Driver:

DOS-2.11 or higher

Size:  128k bytes of expanded memory and 14k bytes of

conventional memory

7.5           ACCENT-1600

SPECIFICATIONS

Parallel Bus:

Toshiba Proprietary Bus

Slot:

Internal Modem slot

Address:

3EE-3EF Hex

Interrupt:

IRQ2 for PC/XT and IRQ9 for PC/AT

Power:

All CMOS circuits, minimum power usage

Load:

2 CMOS load/bus line maximum

Audio Output Power:

0.5W Maximum, 8-Ohm or 4-Ohm Speaker

Physical Dimensions:

5.0" x 4.5" x 0.5" (Maximum component height)

Weight:

0.5 lbs.

Software Device Driver:

DOS-2.11 or higher

Size:  128k bytes of expanded memory and 14k bytes of

conventional memory

7.6           ACCENT-XE

SPECIFICATIONS

Parallel Bus:

Toshiba Proprietary Bus

Slot:

Built-in Modem slot

Address:

2F8-2FF Hex

Interrupt:

IRQ3

Power:

All CMOS circuits, minimum power usage

Load:

2 CMOS load/bus line maximum

Audio Output Power:

0.5W Maximum, 8-Ohm or 4-Ohm Speaker

Physical Dimensions:

2.1" x 5.4" x 0.4" (Maximum component height)

Weight:

0.3 lbs.

Software Device Driver:

DOS-2.11 or higher

Size:  128k bytes of expanded memory and 14k bytes of

conventional memory

LIMITED WARRANTY

We warrant, for a period of one year from the date of shipment to the

original purchaser, that all Accent models are free from defects in

workmanship and materials under normal use  and service, provided the

equipment is used in accordance with the User's Manual.  This Warranty

shall be void if the equipment is modified, the equipment's case (if any) is

opened, or as mentioned elsewhere in the user's manual.  If any failure to

conform to this Warranty appears during the one year period, we will

correct the defect by either suitable repair or replacement, at our option

and at our expense, if you return the product, transportation prepaid, to

Aicom Corporation, 1590 Oakland Road, Suite B112, San Jose, Ca. 95131.

We will pay the cost of returning the repaired or replaced equipment to

you.  The equipment must be returned with proof of purchase in its

original (or other adequate) packaging to the point of purchase or, by prior

arrangement with Aicom Corporation, to Aicom Corporation.

Aicom Corporation reserves the ultimate authority to determine what

constitutes warranty repair.  If it is determined that the product is not

under Warranty, it will be repaired using our standard rates for parts and

labor.  Aicom Corporation advises you to make arrangements with Aicom

Corporation directly or with your Aicom authorized vendor prior to your

returning any equipment for repair.

The above warranty is the only warranty authorized by Aicom

Corporation.

THE ABOVE WARRANTY STATES THE PURCHASER'S EXCLUSIVE

REMEDY FOR ANY BREACH OF AICOM CORPORATION'S

WARRANTY AND FOR ANY CLAIM, WHETHER SOUNDING IN

CONTRACT, TORT OR NEGLIGENCE, FOR LOSS OR INJURY CAUSED

BY THE SALE OF THIS PRODUCT.  WITHOUT LIMITING THE

GENERALITY OF THE FOREGOING, AICOM CORPORATION SHALL

IN NO EVENT BE RESPONSIBLE FOR ANY LOSS OF BUSINESS OR

PROFIT, DOWNTIME OR DELAY, LABOR, REPAIR, OR MATERIAL

COSTS, INJURY TO PERSON OR PROPERTY OR ANY SIMILAR OR

DISSIMILAR CONSEQUENTIAL LOSS OR DAMAGE INCURRED BY

PURCHASER, EVEN IF AICOM CORPORATION HAS BEEN ADVISED

OF THE POSSIBILITY OF SUCH LOSSES OR DAMAGES.

Some States do not allow the exclusion or limitation of incidental or

consequential damages or limitations on how long an implied warranty

lasts, so the above limitation or exclusion may not apply to you.

This Warranty shall not be applicable to the extent that any provision of

this Warranty is prohibited by any Federal, State or Municipal Law which

cannot be preempted.  This Warranty gives you specific legal rights, and

you may also have other rights which vary from State to State.