DEVICE DRIVER FOR GOLD SYNTHESISERS

The file GOLDDEV.SYS is a DOS device driver that allows the Gold

series of synthesisers the emulate an ACCENT PC synthesiser. This

facility enables screen readers such as Flipper or Soft Vert to use

the Gold PC, Gold SA, or Gold Toshiba synthesisers as speaking

devices.

Note even if the synthesiser is SA Stand alone, It still

looks as if it is an internal ACCENT PC synthesiser to the driver.

INSTALLATION

1.   Copy the file GOLDDEV.SYS to your C:\BIN directory, or some other

directory of your choice.

2.   Edit your CONFIG.SYS to include the line: DEVICE=C:\BIN\GOLDDEV.SYS

If the driver was copied to some directory other than \BIN, the

CONFIG.SYS must be set accordingly.

3.   If your screen reader requires another device driver, e.g. ACCENT cards

may use SPKACNT.DVC, then the CONFIG.SYS should be edited to remove

that device driver from loading.

4.   Reboot your computer.

5.   Run your screen reader as if the synthesiser was an ACCENT PC.

The driver auto detects all type of Gold synthesisers. If you have

both a PC card and a SA, the PC card will be used in preference.

If you have a SA, the driver will auto detect the com port the

synthesiser is connected to.

Optional command line parameters will force the driver to look for

a SA on either COM1 or COM2, to specifiy the baudrate of the serial

port used, and to corce checking of the UART baud rate..

To specify the serial port for an SA the line in the config.sys is

DEVICE=C:\BIN\GOLDDEV.SYS 1 for COM1 or

DEVICE=C:\BIN\GOLDDEV.SYS 2 for COM2.

To specify the speed of the serial port, the line in the config.sys

is

DEVICE=C:\BIN\GOLDDEV.SYS 9600                     for 9600 baud or

DEVICE=C:\BIN\GOLDDEV.SYS 38400                    for 38400 baud.

To specify Buad rate checking use a F or f command line parameter eg

DEVICE=C:\BIN\GOLDDEV.SYS 1 f for COM1 or

DEVICE=C:\BIN\GOLDDEV.SYS 38400         2 f for 38400 baud on com2.

This switch is only meaningful on the SA synthesiser.

Baud rate checking is useful when certain screen readers dynamically

change the serial port baud rate from the value set by the device

driver. It checks the current baud rate setting in the UART, and

if it has changed from the driver initial value, resets it back to

the driver initial value.

All paramters may appear together. The default speed is 38400 baud.

Command line parameters other than 1, 2, 9600, 38400, f or F are invalid.

When the driver has located a synthesiser, it will announce its

presence with spoken message. If a synthesiser is not found, no

message can be spoken, but a message willl appear on the screen.

NOTES FOR USING THE DRIVER WITH SOFT VERT Release 4.5

Soft Vert must be installed for an ACCENT PC. To do this run the

INSTALL program on the Soft Vert disk with the parameters

"ACCENTPC 1" or "ACCENT 2". The command line is "INSTALL ACCENTPC 1"

or "INSTALL ACCENT 2".

The "1" or "2" specifies com1 or com2, and although the synthesiser is

an internal type, the parameter is necessary as Soft Vert redirects

the output from the com port specified to the output device.

The INSTALL program will create a file called GOSV.BAT which is

usually run to start Soft Vert. The contents of GOSV.BAT (assuming

the "1" in the INSTALL command line) is:

MODE COM1:9600,N,7,1,P

ptkem C1L3

COPY sinit.txt com1

sv /1

vu accent

This file should be edited as follows.

1. Place "REM " at the start of the first line.

2. place "/b " after the word COPY.

The final result is:

REM MODE COM1:9600,N,7,1,P

ptkem C1L3

COPY /b sinit.txt com1

sv /1

vu accent